A0986.000X BOX ASSEMBLY

1.0	PURPOSE AND SCOPE

This document describes the assembly of all sizes of boxes used for microphones and the artificial mastoid.

2.0	AFFECTED DEPARTMENTS

Manufacturing

3.0	REFERENCE DOCUMENTS

Bill of Materials for A0986.0001, A0986.0002, A0986.0003, or A0986.0004

4.0	RESPONSIBILITIES & AUTHORITY

The person assembling the boxes has the following responsibilities and authority:

- Verify compliance of the product under test to specifications.

- Communicate concerns to the Supervisor of Quality Assurance.

- Request management review of product Concerns.

5.0	DEFINITIONS

None

6.0	SAFETY PRECAUTIONS

Use caution when working with hot glue gun.

7.0	EQUIPMENT & MATERIALS

Hot glue gun

Materials found in Bill of Material

8.0	INSTRUCTIONS

- Take the box (0986.0014) and open the lid.  The part with the latch is the bottom.

- Spread a thin line of hot glue around the inside perimeter of the bottom of the box.

- Press in foam piece M0986.0006

- Spread a thin line of hot glue on top of this piece of foam as in step 2.

- Press in foam piece M0986.0001, M0986.0002, M0986.0003 or M0986.0004 depending on the type of box that is being fabricated.

- Turn the box over and spread a thin line of hot glue around the inside perimeter of the top of the box.

- Press in foam piece M0986.0005.

9.0	INSPECTION

Make sure that foam is secure in box.

Check for “strings” of hot glue and if found, remove.

Clean off any fingerprints from the outside of the box and check for general good appearance.

10.0	RECORDS

No records are generated as part of this process.

11.0	DISTRIBUTION

Manufacturing

Engineering

12.0	ATTACHMENTS

None

13.0	REVISION HISTORY

| DCO # | REV | DATE | INITIALS | CHANGES MADE |
| --- | --- | --- | --- | --- |
| 318 | A | 7/8/01 | SM | Initial Release |