PSA040/PSA043 Battery Charger Work Instructions

# PURPOSE AND SCOPE

The purpose of this document is to provide the detail and instruction necessary to build up a PSA040 or PSA043 battery charger.

# AFFECTED DEPARTMENTS

- Production

# REFERENCE DOCUMENTS

- NA

# RESPONSIBILITIES & AUTHORITY

The technician has the following responsibilities and authority:

- Verify compliance of the product under test to specification.

- Troubleshoot and correct product as required.

- Communicate concerns to Supervisor or Quality Assurance.

- Request management review of product concerns.

- Use proper ESD protective equipment.

# SAFETY PRECAUTIONS

- Follow general electrical precautions for working with energized, low voltage circuits.

# EQUIPMENT & MATERIALS

- Digital volt-ohm meter.

- Soldering iron on a LEAD FREE station.

- Heat gun.

- Engineering ruler with 0.1” increments.

- Safety glasses.

- QTY 1.0, 0195.0061, BATT CHARGER, LD ACID, 12V 3A BARE WIRES. (PSA040)

or QTY 1.0, 0195.0074, BATT CHARGER, LITHIUM, 12V 3A BARE WIRES. (PSA043)

- QTY 1.0, 1212.0022, ANDERSON POWERPOLE RECEP 15A RED.

- QTY 1.0, 1212.0023, ANDERSON POWERPOLE RECEP 15A BLACK.

- QTY 2.0, 1212.0025, ANDERSON POWERPOLE PIN CRIMP 15A.

- QTY 1.0, 1212.0026, ANDERSON POWERPOLE LOCK POST SINGLE.

- 7050.0022, HEAT SHRINK BLK 0.375 ID W/ ADHSV 3:1 (PSA040)

or 7050.0033, HEAT SHRINK, WHT 0.25” ID 2:1 NO ADHSV (PSA043).

- QTY 1.0, 3255.0007, LASER PRINT. SELF LAMIN. LABEL.

# INSPECTION

- Inspect battery charger body and cabling to ensure there is no damage to it.

- Ensure that the model number on the charger body is “1205SRJ” for PSA040 or “Li1205SRJ” for PSA043.

# Assembly Instructions

- Cut wires so length is 12”

- Strip back the sheath by 2.5” and cut the green wire off at the end of the sheath.

- Strip the wires back 0.25”.

- Add heat shrink (7050.0022 for PSA040 or 7050.0033 for PSA043) as shown in figure 1.

Figure 1:  Prep work

- Crimp the pins to the bare wires.

- Assemble the Power Pole housing using the black housing, red housing, pin crimps, and locking post.

- The positive line (white) goes to the red housing; the negative line (black) goes to the black housing.

- Ensure the pin is pushed far enough into the connector to have it click into place.

- Label wire using 3255.0077 with text: “PSA040 (LEAD ACID)” or “PSA043 (LITHIUM)”. See Figure 3.

Figure 2:  Pin Alignment

Figure 3: Finished Assembly

# Electrical Testing

- Power up battery charger.

- With a voltmeter, verify that output polarity is correct and that the output voltage meets the rated volts DC.

# Records NA

# DISTRIBUTION

Instruction is available online in the “Power Supplies” folder in the “Technician” work instruction folder in Document Control.

# REVISION HISTORY

| DCO # | REV | DATE | INITIALS | CHANGES MADE |
| --- | --- | --- | --- | --- |
| ECO 4568 | A | 5/12/2017 | DWA | Initial Release |
| ECO 4620 | B | 8/22/2017 | DWA | Changed from 0195.0070 to PSA040 |
| ECO 4629 | C | 10/9/2017 | DWA | Corrected the connector orientation |
| ECO 4643 | D | 11/2/2017 | DPW | Added length requirement to wires |
| ECO 4870 | E | 8/12/2019 | DCW | Added PSA043 to instructions |