PE2016-LD Safety Rules and Procedures Manual

SECTION 1: PURPOSE

1.1    This document defines the rules and procedures to be followed to ensure the safety of the

employees of Larson Davis a division of PCB Piezotronics, Inc. (LD).

SECTION 2: SCOPE/CONTENTS

2.1    This document outlines general and chemical safety rules and responsibilities for the Company

and Safety Committee.

2.2    List of Sections

TITLE                                                                                                                                          	 PAGE

1       PURPOSE  ……….………………………………………………………………………………………..…..1

2       SCOPE/CONTENTS  …………………………………………………………………………………….......1

3       APPLICABLE STANDARDS AND SPECIFICATIONS  ………………………………………………..…2

4       GENERAL SAFETY RULES AND RESPONSIBILITIES  ………………………………………………...3

5       SAFETY HARNESSES AND LADDERS …………………………………………………………………...4

6       CHEMICAL HANDLING RULES  …………………………………………………………………………...6

7       GENERAL CHEMICAL STORAGE RULES  ……………………………………………………………….9

8       CHEMICAL DISPOSAL RULES  …………………………………………………………………………..10

9       FIRST AID FOR CHEMICAL EXPOSURES…….………………………………………………………..11

10     CHEMICAL SPILL CLEAN-UP  ……………………………………………………………………………12

11     POWERED LIFT TRUCKS  ………………………………………………………………………….…….17

12     ELECTRICAL/MECHANICAL SAFETY POLICY  ………………………………………………….…….20

13     PIPELINE IDENTIFICATION  ………………………………………………………………………….…..24

14     FIRE  ……………………………………………………………………………………………………….…25

15     VENTILATION MEASUREMENT PROCEDURES  ………………………………………………….….27

16     COMPRESSED GAS LEAKS ………………………………………………………………………….…..27

17     CUTTING, WELDING, AND BRAZING [HOT WORK] ……………………………………………….….29

18     GENERAL SAFETY ORGANIZATIONS ………………………………………………………………….30

19     SAFETY & HAZARDOUS WASTE TRAINING……………………………………………………….…..33

20     CHEMICAL TERM DEFINITIONS  ………………………………………………………………………..35

21      PERSONAL PROTECTIVE EQUIPMENT DEFINITIONS ……………………………………………..35

22      PROTECTIVE EQUIPMENT GENERAL INSTRUCTIONS ……………………………………………37

23      SAFETY EYEWASH STATIONS, AND EYEWASH BOTTLES ………………………………..……..38

24      CHEMICAL APRONS AND SUITS ……………………………………………………………………….40

25      GLOVES ………………………………………………………………………………………………….….40

26      EYE PROTECTION …………………………………………………………………………………….…..41

27      FOOT PROTECTION ………………………………………………………………………………….…..42

28      HEAD PROTECTION ……………………………………………………………………………….……..43

29      HEARING CONSERVATION PROGRAM ……………………………………………………….………43

30      RESPIRATORY PROTECTION ………………………………………………………………….……….44

31      PROTECTIVE EQUIPMENT STORAGE ………………………………………………………….……..49

32      ERGONOMICS ………………………………………………………………………………….………….49

33      HAND AND POWER TOOL SAFETY ………………………………………………………….………...51

34      BLOODBORNE PATHOGENS …………………………………………………………………..………..52

35      LASERS ………………………………………………………………………………………….………….54

36      HAZARD COMMUNICATION STANDARD ………………………………………………………..…….54

FORMS ……………………………………………………………………………………………………..……….56

SECTION 3: APPLICABLE STANDARDS AND SPECIFICATIONS

3.1          LARSON DAVIS HAZARDOUS WASTE DISPOSAL REGULATIONS

3.2          AMERICAN FIRE PROTECTION ASSOCIATION

3.3          29 CFR 1910, OSHA STANDARDS FOR THE WORKPLACE

3.4          AMERICAN NATIONAL STANDARDS INSTITUTE

3.5          UTAH CLEAN AIR ACT

SECTION  4: GENERAL SAFETY RULES AND RESPONSIBILITIES

4.1         Wear the safety equipment required for the hazardous situation.

4.2         No smoking in any areas.  Compliance to Utah Clearn Air Act.

4.3         Do not operate equipment under the influence of alcohol or drugs.

4.4         Drink from purified water dispensers in the breakrooms only.

4.5         Keep all areas clean and orderly.

4.6         Dispose of rubbish in the appropriate containers.

4.6.1 Metal waste containers, with self-closing lids, shall be identified as to waste, i.e., flammable.

4.7         Read and understand all instructions and applicable specifications before beginning a job.

4.8         Read and understand all labels before using any materials.

4.9         Treat all unknown liquids as hazardous materials, i.e., flammable, corrosive, oxidizers, or poison.

All containers in Work areas need to be appropriately labeled.

4.10       Never place your hands, feet, fingers, etc., into machinery or unknown liquids.

4.11       No rings or loose-hanging jewelry may be worn by persons working with chemicals, exposed

electrical or mechanical equipment.

4.12       No running.

4.13       No horseplay or fighting.

4.14       Never tamper with the fire extinguishers.

4.15       Do not loiter in areas not required by your job.

4.16       Do not attempt to repair facility electrical equipment unless it is part of your job.

4.17       Perform the job exactly as required. If you have any questions, ask your supervisor.

4.18       Report all injuries to your supervisor or Safety Manager immediately.  Fill out Incident Report.

4.19       Report any exposure to a known or suspected toxic chemical to your supervisor,

or Safety Manager immediately.

4.20       Report all potential safety hazards to your supervisor or Safety Manager.

4.21       Always wash your hands before eating after leaving the work area.

4.22       Always maintain good work-safety habits.

4.23       Know the location of eye stations in your area, if applicable.

4.24        If you work with chemicals, never touch your face or any part of your body with gloved hands.

This prevents the possible transfer of chemicals to skin.

SECTION  5: SAFETY HARNESSES AND LADDERS

5.1        Safety Belts and Safety Harnesses

5.1.1  Safety belts and safety harnesses reduce serious injuries from falls. A safety belt or safety

harness with a lifeline manned by another employee is required when a serious fall hazard

is a possibility.

NOTE: Lifelines provide a means of rescue if an accident occurs.

5.1.2  Safety belts, safety harnesses, and lifelines must be inspected before each use.

5.1.3  Safety belts and harnesses must be used with as little slack in the lifelines as possible.

5.2       Portable Ladder Safety

5.2.1    This standard ensures the proper selection, safe use, and correct maintenance of job-rated

portable ladders.

5.2.2    Areas of Responsibility

5.2.2.1   The Safety Manager has primary responsibility for portable ladder selection and

procurement.

5.2.2.2   It is the responsibilty of workers using the ladders to see to it that the ladders are

- safely stored and properly maintained. Under no circumstances should a

- damaged ladder be used.

5.2.2.3   The Safety Manager is responsible to see that instructions are given to

employees in the safe use of ladders and how to perform safety checks.

5.2.2.4    The employee using the ladder has the responsibility to follow safety practices

outlined in this paragraph and to report all broken or unsafe ladders.

5.2.2.5   The purchasing agent has the responsibility to purchase only ANSI-approved,

job-rated portable ladders.

5.2.3      Deviations from the requirements of this standard will not be permitted, except where

stringent local regulations exist.

5.2.4      Industry recognizes the following general classes of ladders:

    - Type 1 INDUSTRIAL: heavy-duty, for users requiring not more than a 250-pound load.

    - Type 2 COMMERCIAL: medium duty, requiring not more than a 225-pound load.

    - Type 3 HOUSEHOLD: light duty, requiring no more than a 200-pound load capacity.  These ladders are not to be used with stages or planks.

5.2.5       Job rated is a term used to describe the process of selecting a ladder to meet the

- requirements of the job by specifying one of the three basic types.

5.2.6    The following requirements are given for any type of ladder used at LD.

523.6.1   All metal, wood, or reinforced plastic portable ladders must meet the safety

requirements of the American National Standards Institute (ANSI) and the

Occupational Safety and Health Act.

5.2.6.2   The selection of a ladder should account for not only the user’s weight, but any

added weight, such as tools and materials that may exert extra pressure on the

ladder. Type 1 ladders are recommended for most maintenance applications.

5.2.6.3   Ladders must be without structural defects and in good condition. They must be

clean, moving parts must operate freely, and permanently fixed parts must be

sound. Defective ladders must be taken out of circulation and tagged

DANGEROUS, DO NOT USE until repairs are made. Badly damaged ladders must

be destroyed.

5.2.7     When using any type of ladder, abide by the following safe practices:

5.2.7.1   Ladders must not be modified and must be used only for the purpose for which

they were designed. Employees should never reach over their head for a load.

5.2.7.2   Only wood and reinforced plastic ladders will be used for electrical work.

5.2.7.3   Portable metal ladders must be legibly marked with signs reading CAUTION, DO

NOT USE AROUND ELECTRICAL EQUIPMENT.

5.2.7.4   Ladders must not be placed in front of doors opening towards the ladder, unless

the door is blocked open, locked, or guarded.

5.2.7.5   Ladders must not be placed on an unstable base to gain additional height.

5.2.7.6   The top two steps of ladders must not be used as steps or working platforms.

5.2.7.7   Always face the ladder when descending.

5.2.8     Ladders should be stored in such a way as to provide ease of access and safe withdrawal

for use. Ladders should not be exposed to excessive stress, heat, or dampness when

stored. Ladders should be stored against unauthorized use.

5.3.8.1   Contractors working on LD property must furnish their own ladders,

unless trained in the proper use of ladders at our site.

5.2.9     The Safety Committee and ladder users must inspect the ladders in the following manner:

5.2.9.1    Ladders must be inspected every month by the safety inspection team.

5.2.9.2    Ladders must be inspected by the user prior to use and defects noted. The ladder

must be tagged and removed from service until fixed or destroyed.

5.2.10   The inspection criteria for portable ladders follows:

5.2.10.1   Ladders must be free of grease and oil.

5.2.10.2   Wood ladders must not be painted so as to obscure inspection of the wood grain

or hide flaws. They should be painted with a clear spar varnish.

5.2.10.3   Ladders exposed to corrosive materials must be cleaned and rinsed.

5.2.10.4   All metal ladders must be equipped with nonskid feet. Wooden ladders must

have feet in good repair with no splinters or cracks.

5.2.10.5   Ladders should be in good repair and stable, not endangering users in any way.

5.2.11   The following references are cited in the above standard:

5.2.11.1   Safety Requirements for Portable Metal Ladders, ANSI A14.2-1981, American

National Standards Institute.

5.2.11.2   Safety Requirements for Portable Wooden Ladders, ANSI A14.1-1981,

American National Standards Institute.

5.2.11.3   Safety Requirements for Portable Reinforced Plastic Ladders, ANSI A14.5-1981,

American National Standards Institute.

5.2.11.4   Portable Ladders, Data Sheet 1-665-Rev. 82, National Safety Council.

5.2.11.5   Code of Federal Regulations, Title 29, Chapter XVII, 1910.25 and 1910.26.

SECTION  6: CHEMICAL HANDLING RULES

6.1     Ventilation must be adequate to prevent excess fumes from forming in the air.

6.1.1      Aways open the vent and turn on the exhaust fan before using any chemical. The vent fan

must be allowed to operate for twenty minutes after chemical usage has been completed.

6.2     Use only labeled containers and follow the directions on the label. Caps must be left on all bottles.

6.3     All chemical containers must be clearly labeled as to their content and chemical properties.

6.3.1  Refer to the Material Safety Data Sheets for additional first aid and exposure information,

located in your work area or the Safety Manager’s office.

6.3.2   First aid treatment for exposure to a chemical can be found in the "First Aid" major

paragraph of this specification and in the MSDS.

6.4      All containers must be labeled with contents.

6.5      Do not taste or sniff any chemicals, solvents or processing solutions.

6.5.1  Most acids give off fumes that might be toxic. In case of excess inhalation, notify your

supervisor or immediately.  Organic chemicals can have systemic effects.

6.6      Do not heat, mix or prepare chemicals without instructions or knowledge of what should be done.

6.7      Do not permit any unattended hazardous chemical to be heated, exposed or outside the fume hood

during periods where no production is planned or when no one is available for monitoring.

6.8      Flammable liquids and/or containers not being used in production shall be kept in a flammable

cabinet or in flammable metal safety cans.

6.8.1 Small quantities of solvent (less than 1000ml) may be used in minute quantities from a plastic

squirt bottle or aerosol containers.

6.9      If you are uncertain what any spilled liquid is, treat the spill as if it were a hazardous material.

6.9.1 See the spill control procedures in the "First Aid" major paragraph of this specification.

6.10    Do not allow the chemical to contact your body or clothing. Always wear the proper safety

equipment, do not inhale chemical vapors, and keep all ignition sources away.

6.11    ALWAYS check for sources of ignition and ground containers before pouring flammable liquids.

6.11.1 Sources of ignition may include the following:

## Sparks from motors, tools, static electricity, etc.

## Open flame from matches, cigarettes, etc.

## Heat from equipment, chemical reaction, etc.

6.12     Always add acid to water. Never add water to acid.

6.12.1 This is often referred to as the "AAA rule."

6.12.2  Acids should be poured slowly into an approved acid drain while the water is running.

6.13     Never transfer any chemical into a container that is not specifically designated for that chemical.

6.14     Always open chemical containers SLOWLY to allow excess pressure or vapor to escape.

6.15     Never puncture the lid or cap of an acid bottle.

6.16     Never pour hydrofluoric acid into glass or metal containers.

6.17     Never mix chemicals unless the chemical reaction is known or the procedure is specified.

6.18     While being carried, lifted or poured, bottles of chemicals must be supported at the base and

neck.

6.18.1 The neck is the weakest part of the bottle. A break at the neck of the bottle could spill the

entire contents on the carrier or on those working near the carrier.

6.19     Acids, solvents, and other hazardous materials in glass bottles must always be transported in

plastic safety buckets or specially designed carts when being moved anywhere in the facility.

6.20     Acids and solvents must be transported separately.

6.20.1 This prevents accidental mixing of chemicals that produce a violent reaction.

6.21     When transporting chemicals on carts, always fill the bottom tray first to prevent the cart from

becoming top heavy and tipping over.

6.22     Cart and Truck Design (General)

6.22.1 Carts and trucks shall be designed to provide a stable base for the commodities to he

transported and shall have a means of restraining containers against dislodgment.

Carts and trucks shall be provided with a device which will enable the operator to safely

control movement by providing stops or speed reduction devices. Power carts and trucks

shall be approved for Class I, Division 1 or Class I, Division 2 hazardous locations. Internal

combustion engines shall not be used.

6.23     Liquid Transporters

6.23.1 Construction materials for carts or trucks utilized for flammable or combustible

liquids shall be not less than 0.0478-inch (18 gage) steel or approved materials having a

Class 1 flame-spread rating. Exceptions to this rule are materials utilized for casters,

gaskets, trim, and drains.

6.23.2  Liquid carts and trucks shall be designed so that the contents to be transported will be fully

enclosed.  They shall be capable of containing a spill from the largest single container

transported with a maximum individual container size of 5 gallons.

6.23.3  Size

6.23.3.1  Neither the length nor width of a cart shall exceed one-half the width of existing

corridors. The capacity of carts and trucks used for transportation of chemicals

shall not exceed (a) 55 gallons for liquids, and (b) seven cylinders for gases.

6.23.4   Identification

6.23.4.1  Carts and trucks shall bear a marking indicating the contents therein.

Noncompatible materials shall not be transported on the same cart or truck.

6.24         pH Papers

6.24.1 pH papers must be provided in areas where acid or base materials are utilized

NOTE: pH papers are used to identify unknown liquids, such as acids or bases.

6.24.2  Supervisors who have employees that may be exposed to acidic or

basic materials will ensure that pH papers are provided to identify unknown liquids.

6.24.3  To use pH paper, remove a strip of paper from the box and place the strip into the

unknown liquid. When the strip changes color, compare the color to the color chart on the

pH paper box to determine the strength and type of material.

6.24.4  The following pH values are representative of the various pH values:

pH 0-2: strong acid, very corrosive and dangerous

pH 2-5: mild acid, slightly corrosive (Coca Cola is pH 4)

pH 5-9: effectively neutral (pH 7 is exactly neutral)

pH 9-12: mild base, slightly caustic

pH 12-14: strong base, caustic and dangerous

SECTION  7:  GENERAL CHEMICAL STORAGE RULES

7.1     All  chemicals must be marked with precautionary labels.

7.1.1     All Supervisors must ensure that all chemical containers in their areas are properly labeled.

7.1.2     Employees should always handle the chemicals according to the instructions on the labels.

7.2     When in storage, all hazardous chemicals must be separated into the following designated

categories, with the three types segregated from each other. Under no circumstances shall the

following chemical categories be stored adjacent to each other in the same cabinet or on the same

shelf. Corrosives, Oxidizers, Solvents

7.2.1   Chemicals in bottles must be stored in cabinets or on shelving when not in use. The work

area is not a long term storage area, and if the chemical is not going to be used during a

work shift, it should be put away.

7.2.2   Open shelving must have an edge spacing of 1" minimum to safely contain bottles.

7.2.3    Always store flammables in the proper cabinets, specially designed for this purpose.

7.3     In all fabrication, process, and research areas, hazardous chemical materials must be stored at eye

level (five feet two inches) or below.

7.3.1 This will minimize the risk of chemicals being spilled on the skin or in the eyes.

7.4     Any flammable chemicals not being used in a work area must be kept in a flammable storage

cabinet.

7.4.1  Gases utilized by a work area will be supplied only on an as needed basis. No long term

storage within work areas will be allowed.

7.5     Gas cylinders will be handled and stored by authorized technicians, trained for this purpose.

7.6     Gas cylinders must be properly capped, chained, labeled and stored in an upright position.

7.6.1  It can be very dangerous to lay a cylinder on its side, especially Acetylene.

7.6.2   If a gas cylinder is knocked over and its top is broken, dangerous gases could be released

and the extremely high pressure present could propel the cylinder like a rocket.

7.7     Gas cylinders must be transported while secured in proper cylinder carts.

7.8     Gas cylinders must be stored in dry, cool (below 80 degrees) ventilated areas.

7.9     Open containers of hazardous materials must be kept under exhaust hoods or in specially

ventilated areas.

SECTION  8: CHEMICAL DISPOSAL RULES

8.1     When empty, ALL acid containers shall be drained completely and then thoroughly rinsed three

times (triple rinsed) before disposal, in accordance with legal requirements.

8.1.1 Acid bottles must also be rinsed on the outside.

8.2     Acid bottles, after being completely emptied and triple rinsed, must not be used to store drinking

water or food.

8.3     Solvent bottles, after being completely emptied, must be recapped and disposed. Under no

circumstances shall these bottles be rinsed out with water. This is to prevent solvent contamination

from going down the drains.

8.4     Employees who dispose of the empty bottles in the dumpster must wear the following safety

equipment:

Safety glasses (or face shield) Protective gloves (acid resistant) Apron or jacket (acid resistant).

Always use the proper methods to dispose of hazardous materials.

8.5.1  Waste acids must be neutralized before pouring them down the drain for disposal. The Safety

Manager should be consulted before any disposal takes place, as certain materials must be

treated as hazardous waste and disposed of approprately.

8.5.2  Solvents must never be poured down the water drain.

** CAUTION **

MIXING OF ACIDS AND SOLVENTS CAN CAUSE CHEMICAL REACTIONS AND FIRES. ACID

DRAINS ARE TO BE USED ONLY WHEN ACIDS ARE DILUTED WITH WATER ON A 1:10 OR

MORE DILUTION. THE USE OF WATER DRAINS FOR SOLVENT DISPOSAL IS PROHIBITED.

8.6     All chemicals designated as “controlled waste material" will be segregated for proper disposal. See

Document #WI20.0003, Hazardous Waste Disposal Regulations. These must not go down drains.

8.7     Flammable waste must be placed in metal waste containers (with self-closing lids) and identified as

to type of waste. Safety department approval is required.

8.7.1   Spill wipes, paper towels or other absorbent materials saturated with a flammable liquid must

be placed in metal waste cans labeled FLAMMABLE WASTE.

8.7.2   Other wastes could cause a dangerous chemical reaction if combined with the flammable

waste.

8.7.3   It is the responsibility of the person creating the waste to ensure that the hazardous waste

containers are properly labeled.

SECTION  9: FIRST AID FOR CHEMICAL EXPOSURES

** CAUTION **

IN THE EVENT OF ANY ACCIDENT, NOTIFY YOUR SUPERVISOR IMMEDIATELY.

WORKMAN’S COMPENSATION WILL NOT APPLY IF YOU DO NOT REPORT ALL ACCIDENTS.

9.1    Chemical Burns (All Types)

9.1.1  Rinse the area thoroughly for 15 minutes with water (H20).

** CAUTION **

ACIDS AND CAUSTICS MAY SOAK THROUGH CLOTHING AND ATTACK THE SKIN. RINSING

THE CLOTHING WILL NOT DILUTE THE ACID ON THE SKIN. IT IS IMPORTANT TO REMOVE

CLOTHES QUICKLY AND DILUTE ANY ACID ON THE SKIN. DO NOT LET MODESTY KEEP

YOU FROM PREVENTING A SERIOUS BURN.

9.1.2  Notify your supervisor and report to the Safety Manager or site Emergency

Medical Technician.

9.1.3  After treatment, fill out an "Incident Report”  and give it to your supervisor. Forms are

available on line (MyPCB.com\, TCS logo, Human Resources, Incident Report), from the

Safety Manager, and Human Resources.

9.2     Thermal Burns

9.2.1  Notify your supervisor or/and report to the Safety Manager for treatment.

9.2.2  Fill out an "Incident Report" and give it to your supervisor.

9.3     Inhalation of Gases or Acid Fumes

9.3.1  Get fresh air immediately. Call the operator or have someone notify, the Safety Manager or

site Emergency Medical Technician who will come and provide treatment.

9.3.2  Fill out an "Incident Report" and give it to your supervisor or Safety

Manager.

9.4      Chemical Contact with Eyes

9.4.1  If chemicals contact the eyes, the eyes should be flushed with water immediately at an eye

wash station.

9.4.1.1 While the eyes are being flushed, hold the eyelids wide open.

9.4.2  The eyes should be flushed for 15 minutes before the employee receives further first aid.

9.4.3  The employee should then go directly to the Safety Manager or site Emergency Medical

Technician for treatment.

** CAUTION**

DO NOT ALLOW ANYONE BUT A MEDICALLY TRAINED PERSON TO REMOVE FOREIGN

BODIES FROM THE EYES.

9.4.4  In all first aid treatments, the supervisor and Safety Manager should be notified and an

“Incident Report" should be filled out by the Supervisor and the victim.

** WARNING **

DO NOT ATTEMPT TO MOVE AN INJURED PERSON WHO IS UNCONSCIOUS OR NOT

MOVING. WAIT UNTIL PROPER MEDICAL ASSISTANCE ARRIVES.

SECTION 10: CHEMICAL SPILL CLEAN-UP

10.1   The supervisor must be informed of any spills.

10.1.1  The supervisor must ensure that there are enough safety materials and

equipment readily available to handle a worst case situation, and notify the Safety Manager.

10.2    Small Spills

10.2.1  Small spills may be cleaned up by a LD technican.

10.2.2  A "small spill" is a spill with a diameter of 6 inches or less.

10.2.3  For a small solvent spill, do the following.

10.2.3.1  Wear protective chemical gloves, plastic apron, and safety glasses.

10.2.3.2  Absorb the spill with spill control pillows.

10.2.3.3  Place the wet spill control pillows in the fireproof  waste container.

** CAUTION **

ONLY CLEAN SPILL CONTROL PILLOWS OR VERMICULITE IS TO BE USED TO

CLEAN A SMALL SOLVENT SPILL.

** CAUTION **

DO NOT DISPOSE OF SOLVENT ABSORBENT IN ANY CANS CONTAINING ACID

WASTE MATERIAL.

10.2.4    For a small acid/corrosive spill, do the following.

10.2.4.1  Wear protective chemical gloves, plastic apron, and safety glasses.

10.2.4.2  Dilute with water and absorb with paper towels.

10.2.4.3  All materials used to clean up small corrosive liquid spills (less than 1 gallon) or

materials used to wipe down acid sinks and clean up small drips are to be

thoroughly rinsed with water before being disposed of in a waste basket. This

includes paper towels, chemical wipes, and rags.

10.2.4.4  Rinse the area with water and absorb with towels.

10.3      Large Spills

10.3.1    If a large spill occurs, technicians must evacuate the immediate area.

10.3.2    A "large spill" is a spill with a diameter greater than 6 inches and a volume up to one

gallon.

10.3.3   The supervisor and Safety Manager must be notified immediately.

10.3.4   Large spills must be cleaned up by at least two people. This is to provide aid should

fumes overcome one individual. Use the procedures outlined below.

10.3.4.1  For "acid spills" do the following: wear protective gloves, apron, and safety

glasses. Neutralize the spill if sodium bicarbonate or equivalent neutralizer is

available. Dispose of the neutralized material in a small plastic bag. Mop up the

residue with a wet sponge or towel. Dispose of the wet sponge in the plastic

bag. Label the bag (as per paragraph 10.5.7).

** CAUTION **

DO NOT DISPOSE OF THE MATERIAL IN ANY CAN CONTAINING SOLVENTS OR

SOLVENT WASTE.

10.3.4.2   If neutralizers are not readily available, wear gloves, apron, and safety

glasses. Absorb the spill using spill pillows, paper towels, or equivalent inert

absorbent. Rinse the spill area thoroughly with water. Dispose of excess liquid

down an acid drain.

** CAUTION **

PEROXIDE SPILLS SHOULD NEVER BE ABSORBED WITH PAPER TOWELS OR

ANY ORGANIC MATERIAL, SUCH AS VERMICULITE, AS A FIRE COULD RESULT.

10.3.4.3  Dispose of the spill soaked absorbent in a polyethylene bag. Label the

polyethylene bag (per paragraph 10.5.11).

10.3.5      For "large solvent spills", use the procedures stated in the "Procedures for Organic

Solvent Spills" paragraph 10.6.

10.4   Major Spills

10.4.1     If a "major spill" occurs, technicians must evacuate the immediate area.

10.4.2     A "major spill" is a spill with a volume of 1 gallon or more.

10.4.3     The following people must be notified in the event of a major spill:

Supervisor, Safety Manager, and site Emergency Medical Technician. A

major spill cleanup should be medically supervised.

10.4.4    Major spills must only be cleaned up by those members of the Safety Committee specially

trained for large spill cleanup.

10.4.5    The following safety equipment MUST be worn when cleaning up a major spill:

-Chemical protective gloves

-Safety glasses or face shield (or both)

-Respirator with yellow cartridges (or others designated by the Safety Manager)

-Acid-resistant pant/coat or coveralls

-Acid-resistant shoes or shoe coverings

10.5     Procedure for Acid/Corrosive Spills

** CAUTION **

ALL THE SAFETY EQUIPMENT LISTED IN THIS PARAGRAPH MUST BE WORN BY ALL

PERSONNEL RESPONDING TO A CORROSIVE SPILL.

10.5.1   The following materials are required for cleaning up a major spill:

    - pH indicator paper

    - Neutralizing agent (use sodium bicarbonate for acids)

    - Plastic scoop

    - Plastic dust pan

    - Polyethylene disposal bags

    - Absorbent material (spill pillows, paper towels, etc.)

** CAUTION **

IF YOU ARE UNCERTAIN WHAT ANY SPILLED LIQUID IS, TREAT IT AS IF IT WERE

AN ACID SPILL.

10.5.2    Using pH paper, identify the spill as either acid/corrosive or solvent.

10.5.3     Isolate or contain the spill.

10.5.4     Keep all unnecessary foot traffic away from the spill.

10.5.5     Avoid inhaling the acid vapors.

10.5.6     Using a plastic scoop, gently spread neutralizer over the spill, working from the perimeter

inwards.

10.5.6.1  If a neutralizer is not available, absorb the acid using spill pillows or absorbant.

10.5.7     Apply neutralizer until the spill is neutralized. Use pH papers to determine pH level.

** CAUTION **

IT IS NORMAL FOR FOAMING AND GAS FORMATION TO OCCUR DURING THE

NEUTRALIZATION PROCESS.

10.5.8    After the acid has been neutralized or absorbed, transfer the spill material into

polyethylene disposal bags.

10.5.9    Use a wet sponge or towels to mop up the residue.

10.5.10  Put the used sponge in a polyethylene disposal bag.

10.5.11  The disposal bags must be labeled with the following information:

-Type of waste in the bag

-Area where waste was generated

-Responsible supervisor

-Date

10.5.12  Store the filled disposal bags in an acid storage area.

10.6     Procedure for Organic Solvent Spills

10.6.1  Using pH paper, identify the spilled material as either an acid or corrosive.

10.6.2  Isolate or contain the spill. Do whatever is necessary to prevent liquid from running down

water drains or contaminating the ground if outside.

10.6.3  Keep unnecessary foot traffic away from the spill.

** CAUTION **

THE FOLLOWING SAFETY EQUIPMENT MUST BE WORN: CHEMICAL APRON,

GLOVES, SAFETY GLASSES (OR FACE SHIELD), AND RESPIRATOR WITH YELLOW

CARTRIDGE. IF OXYGEN IS BEING DEPLETED BY THE CHEMICAL FUMES, AN AIR

SUPPLIED RESPIRATOR OR AIR PACK MUST BE WORN.

10.6.5  Place towels around the outside of the spill and work towards the center.

10.6.5.1 Spill pillows, vermiculite, or sand may be used instead of paper towels.

10.6.6  As the absorbents become saturated, place them in polyethylene disposable bags.

10.6.7  When the spill is cleaned up, put all the used towels or spill pillows in the polyethylene

disposal bag.

10.6.8  Label the waste bags per Paragraph 10.5.11.

10.6.9  Store the bags in a solvent storage area (or in a fireproof can) until the materials can be

disposed.

10.7     Procedure for Leaking Gas Cylinders

10.7.1  Nontoxic Gases

10.7.1.1  If a nontoxic leaking compressed gas cylinder is discovered, the supervisor of the

area and the Safety Manager should be contacted immediately.

10.7.1.2  Compressed gases which are nontoxic, with the exception of hydrogen, may be

removed from the immediate area. These gases present no health hazard by

themselves, but a leaking cylinder can displace oxygen in a confined space so

great care must be taken when entering the area. The gas cylinder may be

transported to an outside leaking cylinder area by the safest route.

10.7.2  Toxic Gases

10.7.2.1 If a hydrogen or toxic compressed gas cylinder is leaking and conditions dictate,

the immediate area shall be evacuated. Speed is of the essence.

10.7.2.2  The following groups shall be contacted by the area supervisor: Safety Manager,

site Emergency Medical Technician, and the General Manager.

10.7.3  The Safety Manager shall contact the gas vendor following incident assessment. Vendors

have special hazardous leak teams that will respond.

10.7.3.1  Toxic gases and Hydrogen shall only be used in a vented gas cabinet. The leaking

gas cylinder shall remain in the gas cabinet until the using technician establishes the

source of the leak. Proper personal protective equipment shall be worn, such as

SCBAs, air supplied respirators or other equipment required by the Safety Manager.

10.7.3.2  If the leak is discovered in the regulator, piping or fixturing after the regulator, shut

off the supply of gas at the source tank.

10.7.3.3  If the gas is pyrophoric, primary exhausting should be back through the process

equipment, if possible.

10.7.3.5  If the leak is through the stem or diaphragm, the gas cylinder shall remain in the

cabinet and vent into the cabinet. The following steps shall be performed: (1) DO

NOT REMOVE the cylinder into the hallway, room, or outside until such process

has been reviewed and approved by the proper safety personnel. Keep the area

clear and follow directions as given by the vendor or Safety Manager. (2) Even if it

has been determined that the cylinder is leaking through the neck or lower valve

assembly,  where possible, attempt to turn off the cylinder at the valve. (3) If the

cylinder is allowed to be removed, place it in the outside leak area, and label it.

10.7.3.6  The vendor shall be responsible for final deposition of the leaking, toxic cylinder.

SECTION 11: POWERED LIFT TRUCKS

NOTE: LD does not own a power lift truck at this time.  If one is rented or brought onto the site the

following procedure will apply.

NOTE: This major paragraph provides requirements for forklifts and powered pallet jacks.

11.1    Deviations from the requirements of this major paragraph are not permitted except where area

specifications are more stringent.

11.2    Safety Manager Responsibilities

11.2.1  Ensure that each powered lift truck procured for the department satisfies the requirements

according to the American National Standards Institute (ANSI) B56.1-1975.

11.2.2  Ensure that each employee who operates a powered lift truck has completed a safety

program which includes training in the following:

-Driver training and certification

-Preventive equipment maintenance

-Safe operating practices

11.2.3   Post powered lift truck operating rules in a conspicuous place in each area where powered

lift trucks are used.

11.2.4   Ensure that each employee who operates a powered lift truck has been properly trained.

11.2.5   Ensure that each powered lift truck has an attached tag that lists the weight, load capacity,

and other data pertinent to the operation of the powered lift truck.

11.2.6   Ensure that each powered lift truck has an attached tag showing that the truck has been

tested in accordance with nationally recognized testing standards.

11.2.7   Ensure that forklifts are equipped with an overhead guard to provide protection from falling

objects.

11.2.8   Ensure that each powered lift truck is equipped with a working audible warning device such

as a bell or horn.

NOTE: A rotating or flashing light may not be used as a substitute for an audible

device.

11.2.9   Ensure that each powered lift truck is equipped with an audible back-up alarm.

11.2.10 Ensure that powered lift trucks used outdoors on a rough, soft, or uneven surface are

equipped with pneumatic tires.

11.2.11 The Safety Manager ensures that at the beginning of each day, the forklift driver shall

perform an inspection of the equipment, using Form WI20.08, “Forklift Inspection Log”,

kept in the forklift inspection log book, readily available in the work area.

11.2.12 Ensure that a broken or defective forklift is not used or returned to service until all repairs

have been completed.

11.3      Operator Responsibilities

11.3.1  Follow the requirements of this major paragraph for operating a powered lift truck.

NOTE:   A powered lift truck is considered unattended when the operator is more than

25 ft from the vehicle or the truck is out of their sight.

11.3.2  When a powered lift truck is left unattended, do the following:

11.3.2.1  Lower the forks to the ground.

11.3.2.2  Put the drive control in neutral.

11.3.2.3  Set the parking brake.

11.3.2.4  Shut off the power.

11.3.2.5  Remove the key and store in a safe place. This is to prevent unauthorized use by

people who have not been trained in forklift operation.

11.3.2.6  If the vehicle is on an incline, chock the wheels.

11.3.2.7  Never give rides on a forklift or pallet jack.

11.4    Operator Training

11.4.1   The Safety Manager shall certify each powered lift truck operator in accordance with

approved training procedures.

11.4.2   Such certification shall be in force for 1 year.

11.4.3   To receive training for operating a powered lift truck, do the following:

11.4.3.1  Obtain permission from your supervisor.

11.4.3.2  Contact the Safety Manager to schedule training sessions.

11.4.4   The Safety Manager shall ensure proper certification of each powered lift truck operator

through a combination of experience and classroom instruction.

11.4.4.1  The following will be included in classroom instruction:

      - Fundamentals

      - Operating principles

      - Safety rules

      - Vehicle safety checks

      - Proper use of controls

      - Maneuvering skills

      - Selecting loads

      - Stacking loads

      - Moving loads

11.4.5   Evaluate each powered lift truck operator for certification by written exam and observed

skill assessment.

11.4.6   When the certification requirements have been successfully completed, issue a powered

lift truck operating license.

11.4.7  Carry the powered lift truck operating license when operating such vehicles.

NOTE: The powered lift truck operating license may be revoked for disciplinary

reasons, unsafe use of the vehicle, or failure to meet the physical

requirements.

11.4.8  If a load obscures vision, a spotter shall accompany the lift truck operator.

SECTION 12: ELECTRICAL/MECHANICAL SAFETY POLICY

12.1   This policy establishes a uniform method of warning tags and signs for electrical circuits, power

driven equipment and pressurized lines while personnel are working in a position in which activation

of electrical circuit, power or pressure creates an undue hazard.

12.2   Requirements and Exceptions

12.2.1 Electrical/electronic equipment shall be installed in accordance with NEC (National Electrical

Code) 70E, Employee Electrical Safety. Additional information may be obtained in the

following chapters:

70E Part I

    - Installation Safety Requirements - Chapter I

    - Wiring Design & Methods - Chapter II

    - Wiring Methods - Chapter III

    - Specific Purpose Equipment - Chapter IV

    - Hazardous Location - Chapter V

    - Special System Over 600V - Chapter VI

70E Part II

    - General Requirements - Chapter I

    - Personal & Protective Equipment - Chapter II

    - Specific Safety Procedures - Chapter III

    - Lockout/tagout - Chapter IV

70E Part III

    - Safety Related Maintenance Practices - Chapter I-5

    - Portable Electrical Tools -Chapter VI

    - Safety & Protective Equipment - Chapter VII

    - Batteries & Battery Rooms -Chapter IX

12.2.2    Only qualified craftspersons shall work on electrical, mechanical and automatic starting

equipment. Qualified shall mean having training equal to or technical experience and

familiarity with similar equipment and associated hazards. (See Section 12.6 for electrical

qualifications and training requirements.)

12.2.3   Tagout procedures are required on equipment or machines which use electricity as either the

primary source of power or as the control for other sources of power such as water, oil and

gas. Tagout devices should be used while any guard is out of place, while changing any

component of a machine or while repairing any electrically powered machinery or equipment

except:

12.2.3.1  In those cases where equipment is required to be in operation while repairs are

being made (see 12.2.4 and 12.2.4.1).

12.2.3.2  On extremely small-powered drives, such as instrument motors, chart drives, etc.

12.2.3.3  If equipment receives its power through a portable cord, the plug shall be removed

from its receptacle and the cord tagged with a DO NOT OPERATE tag.

12.2.4   Electronic Equipment Exception

12.2.4.1  Performance repairs on equipment is excepted from the tagout requirements only if

all other safeguards, equipment barriers or procedures of equal or superior

protection as necessary to prevent accidental contact are insured.

12.2.5   Tagout Procedures are Required:

12.2.5.1  On electrical control switches, while at work on de-energized electrical lines, where

the maximum voltage exceeds 70 volts.

12.2.5.2  On valves or pipelines containing hazardous substances or pressures to prevent

the unintentional turn-on while under repair or adjustment.

12.2.5.3  On all automatic starting equipment, such as compressors, generators, while under

repair or adjustment.

12.2.5.4  On any electrical work, where the view of a switch is obstructed and the

unintentional activation of that switch may occur.

12.2.6    Equipment Required for Tagout

12.2.6.1  A lockout/tagout tag.

12.2.7   Equipment for Electrical Equipment Exception

12.2.7.1  A suitable portable grounding rod(s).

12.2.7.2  A pair of rubber insulating gloves (ANSI J6.6).

12.2.7.3  Rubber matting (ANSI J6.7).

12.2.7.4  Rubber insulating hood (ANSI J6.2).

12.2.7.5  Rubber insulating blanket (ANSI J6.4)

12.2.7.6  Rubber insulating sleeves (ANSI J6.5).

12.2.7.7  Any other equipment, barriers or procedures to prevent accidental contact.

12.2.8   Disconnect Devices: Electrical/electronic equipment shall be installed in accordance with

NEC and NFPA 70E, Part 2, Chapter IV, and 70E, Part I, Chapter I:F.

12.2.8.1  An individual power isolating device shall have the following characteristics: It will be

convenient to the equipment it controls so as to promote its use, It will be clearly

marked or labeled so as to make it clear which piece of equipment it serves (the

one exception to this rule is where the power connection to the equipment is

obvious such as with a plug).

12.2.8.2  In the case of previously installed equipment or in new installations where a

dedicated energy isolating device is highly impractical, the responsible supervisor

in the area will do the following: Ensure that the electrical service device activating

multiple pieces of equipment is  clearly posted with a warning indicating that the

device activates multiple equipment and the equipment or areas it controls. Ensure

that each electronic craftsperson working in the area is fully trained as to which

isolating devices activate multiple equipment (see Section 12.6).

12.3    Safety Precautions

12.3.1  In all operations or processes that endanger the safety of employees, tagout devices shall

be used wherever possible. Supervisors shall enforce correct tagout procedures.

12.3.1.1  The minimum disciplinary action for a person not following tagout procedures

when it is required will be a written warning.

12.3.2  Tagouts will be accomplished as follows:

12.3.2.1  When work is being performed on electrical circuits or power driven equipment,

the employee performing the work will disconnect the nearest suitable power

isolating device and tag it with his personal tag. If components are capable of

storing an electrical charge the equipment will also be discharged to ground. The

equipment will then be de-activated and de-energized before work begins.

12.3.2.2  Upon completion of the job, the employee shall remove the tag and notify the

responsible person concerned that the equipment is ready for use. Equipment will

not be restarted until checked and all tags, signs have been removed.

12.3.2.3  When two or more crafts are involved, a multiple tag procedure must be used.

12.3.2.4  Each craftsperson shall use their own lockout tag on the multiple device so that the

equipment will not be activated until each craft has removed its tags.

12.3.2.5  Supervisors who wish to make a final check before re-energizing the circuit should

also add their tag in line with the others.

12.3.2.6  Whenever a tag is used, and the tag is at a location separate from the equipment,

DO NOT OPERATE TAG shall be attached to the equipment under repair.

12.3.2.7  All personnel who perform work requiring positive tagout protection must be

issued three lockout tags. Craftspersons are asked not to wear jewelry while

repairing electrical equipment.

12.3.2.8   Where it is impossible to follow tagout procedure, a DANGER tag will be installed

over the control device and immediate steps to correct this condition shall be

implemented. A two person work crew shall be used on all non-tagout of electrical

equipment.

12.3.2.9   Tags will be left on the equipment only during the actual shift when work is being

performed. Any tagouts left on the equipment overnight must be authorized by the

responsible supervisor. Incompleted jobs will be continued to be tagged out with a

DO NOT OPERATE tag and will list those to contact if questions occur.

12.3.2.10  Barriers and signs to denote "DANGER EXPOSED HAZARD" are required to

assure positive control in areas of multiple high hazard exposures.

12.3.3    Equipment Interlocks

12.3.3.1   Safety interlocks and/or required protective paneling shall not be passed or

defeated in any way during repair or adjustment operations unless other

safeguards or equal or superior protection are afforded.

12.4   Two Person Work Crews

12.4.1  Whenever it is impractical to observe a positive tagout procedure then a two person work

crew shall be used.

12.4.2  The one person will act as an observer until the repairs have been completed.

12.5   Unauthorized Tag Removal

12.5.1  Tagouts will not be removed by anyone other than the craftsperson who installed it. The

responsible supervisor may remove a tagout in an emergency situation only after checking

the entire line. Any other person removing tagouts not belonging to that person shall receive

immediate disciplinary action - up to and including termination.

12.6  Training In LD Facility Procedures and Electrical Safety

12.6.1  In all cases, the responsible supervisor will ensure that the training specified below is

accomplished and recorded within the stated time frames.

12.6.2  All persons designated by their supervisors as qualified to work on their area's (electrical)

equipment shall have completed the LD Electrical Safety Training listed below

before they are allowed to work unsupervised on equipment in their area.

12.6.2.1  The LD electrical safety training will involve attending the facility

Electrical Safety Class and passing the examination.

12.6.2.2   When the employee has completed the training and successfully passed the

examination, the safety department will:

- notify the supervisor of his passing the exam;

- post this information in the employee's training file;

- be issued lockout tags.

12.6.3   All contractors entering the LD facility will be instructed in the company’s tagout

policy by the Safety Manager. In addition, the contractor will sign a statement that they were

notified of the facility electrical/safety policy.

12.6.4   To ensure that all employees understand the importance of equipment tagout and warning

signs, this information will be presented during new hire safety orientation.

SECTION 13: PIPELINE IDENTIFICATION

13.1   This major paragraph establishes standards which will minimize hazards associated with

substances piped under pressure, and formalizes a uniform system of pipeline identification. Piping

systems include pipes of any kind, fittings, valves, and pipe coverings. Pipe is defined as a conduit

for the transport of gases, liquids, or semiliquids.

13.2   Responsibilities

13.2.1  It is the responsibility of the site Safety Manager to assure that all facility piping systems

serving the site or supplying area equipment comply with the requirements of this standard.

13.2.2  Deviating from standard requirements is not permitted except in cases where local

regulations are more stringent.

13.4   Requirements

13.4.1 This standard considers the legend to be primary and explicit for content identification.

Positive identification of the content of the piping system will be by lettered legend, giving the

name of the contents in full or abbreviated form. Arrows will be used to indicate flow direction.

Use legend to identify content exactly and give temperature, pressure, and other details as

applicable. Apply legend close to valves, adjacent changes in direction, branches, where

pipes pass through walls or floors, and at 20-foot intervals on straight runs. Place labels on

10-foot intervals for hazardous materials.

13.4.2  Color may be used to indicate whether the materials conveyed in the pipes are hazardous,

safe, or flammable. Legends and flow directional arrows will be printed on the specified color

field. Banding tape, used to end band the legend, will be the same color as the legend.

13.4.3  Place markers on pipes in areas where they will be most visible to employees. When pipes

are located near ceilings or above line of sight, place markers on the bottom of the pipes

where clearly visible. On vertical pipes, place markers on any side where visibility is best.

Letter height must be readable. Color field size should conform to pipe size.

13.5   References

13.5.1  The following references are cited for the above standard:

13.5.1.1  "Code of Federal Regulations," Title 29, Chapter XVII, Parts 1910.144 and 1910.145

13.5.1.2  "Scheme for the Identification of Piping Systems," ANSI A13.1-1975, American

National Standards Institute

13.6.1.3  "Uniform Fire Code," 1985 Edition, Chapter 74, Section 205(g), International

Conference of Building Officials and Western Fire Chiefs Association

SECTION 14: FIRE

14.1   There are three classifications of fires:

- Personal Fire

- Minor Fire

- Major Fire

14.2   In all cases of fire, response must be immediate.

14.3   Response to Personal Fire:

14.3.1 A "personal fire" is a fire on the clothing or hair of a person.

14.4   Immediately call out for help.

** CAUTION **

DO NOT RUN. RUNNING MAY INCREASE THE SEVERITY OF THE FIRE.

14.5  People in the area should immediately assist the victim and contact the receptionist.

** CAUTION **

A PANIC-STRICKEN PERSON MAY HAVE TO BE FORCED TO THE  GROUND TO PUT THE

FIRE OUT.

14.6   When the fire has been extinguished, the person will be evaluated by the Safety Manager or site

Emergency Medical Technician, no matter how slight the burn.

14.7    The "Incident Report” must be completed.

14.8.1 The report form is available from the Safety Manager or Human Resources.

14.9    Response to a Minor Fire:

14.9.1  A "minor fire" is a fire that is contained, is not spreading and presents no IMMEDIATE

threat to the area or to the personnel in the area.

14.9.2  A "minor fire'' can typically be extinguished with a portable fire extinguisher.

14.10   Immediately call out for help.

** CAUTION **

A MINOR FIRE MUST BE CONTROLLED QUICKLY TO PREVENT THE POSSIBILITY OF A

MAJOR FIRE.

14.11  Find another person for assistance.

14.12  The immediate area should be evacuated under the direction of the designated area Safety

Committee member.

14.13  Attempt to put out the fire with a fire extinguisher until relieved by a Safety Committee member.

14.13.1  All supervisors and Safety Committee members will be trained in fire

extinguisher usage.

14.14  Fire extinguishers only discharge for 10-15 seconds. Heat captured in materials on fire can also

lead to a flash back of the fire after it appears that it is out. Use great care in using the fire

extinguisher so that flames will not curl back on the user.

14.15  When the fire is out, notify the Safety Manager.

14.15.1  Used fire extinguishers must never be returned to their mounting brackets. They should be

returned to the Safety Manager to be refilled.

14.16  The Safety Manager must ensure that the portable fire extinguishers are always fully charged and

operational. A monthly survey of every fire extinguisher will be made and guage readings

monitored, using Form PE2016-2, “Fire Extinguisher / Emergency Light Inspection”.

14.16.1 Any extinguisher removed for repair or refilling must be replaced immediately.

14.17   Response to a Major Fire:

14.19.1  A "major fire" is a fire that is large, uncontained and cannot be controlled with a portable

fire extinguisher.

14.19.2  Call out for help and call 9911. Have someone call the receptionist

and report the fire.

14.19.2.1   The Receptionist is to call 9911 and report the fire if the call to the fire

department was not already placed.  An alarm on the receptionist’s consel will

sound when anyone dials 9911to bring in city fire fighters.

The Receptionist will also announce over the company intercom system that

there is a fire evacuation for the building where the incident occurred.

14.19.2.2   All employees must immediately evacuate the incident building and report to

designated fire drill holding areas. Supervisors will perform a head count.

14.19.2.3   All LD Safety Committee members should report to the location of the

incident and take assignments from the first responder or Safety Manager.

14.19.3   The Safety Committee members should confirm that nobody remains in the fire area.

14.19.4   The Safety Committee members will isolate the fire by closing all doors leading to the fire

area. This will inhibit the spread of fire.

14.19.5    Good judgement should be excercised as to whether the fire is best left for professional

fire fighters with extensive fire fighting equipment. If this is the case, all Safety Committee

members should evacuate the area and report to the designated fire drill holding area.

14.19.6    Supervisors and the Safety Manager will maintain employee control in the fire drill holding

areas and ensure that no employee returns to the incident area for personal belongings.

14.19.7    The Safety Manager will interview all individuals directly involved in the incident and

submit an incident report to the General Manager within 24 hours of the fire.

14.19.7.1  A review of the incident report will be made by Management and procedures

instituted to prevent another fire of similar cause.

SECTION 15: VENTILATION MEASUREMENT PROCEDURES

15.1   Hood and process cleaning vat ventilation conformance measurements shall be conducted by the

Safety Manager, in accordance with accepted standards of performance. Such standards are found

in the codes listed below.

  - American National Standards Institute (ANSI); Z9.2-1976

  - National Fire Protection Association (NFPA); 91-1973

  - American Conference of Governmental Industrial Hygienists (ACGIH), Industrial Ventilation Handbook, Vol. 18 (1984)

  - Uniform Building Code (UBC); Section 911(3)

  - Uniform Fire Code (UFC); Section 51.105(d), 51.106(c)

15.2   Survey results will be retained by the Safety Manager.

15.3   Inspections for compliance will be conducted on a periodic basis, not less than annual, or as

requested by LD employees. Results will be logged on Form PE2016-3, “Ventilation Inspection”.

15.3.1  Any ventilation apparatus found to be out of compliance will be placed out of service, a sign

to this effect posted, and the Safety Manager will ensure that repairs are made.                    .

SECTION 16: COMPRESSED GAS LEAKS

16.1   Response to a Detected or Suspected Gas Leak

16.2.1  Evacuate the area. Even non-hazardous gases can present a life threatening situation

because they can displace oxygen in a confined space.

16.2.2  Notify the following:

Safety Manager,  General Manager, Logistics Manager and Production Manager.

16.2.3  Actual events will dictate the order in which Paragraphs 16.1 and 16.2 will be followed.

16.2.4  Any person exposed to a poisonous gas leak without proper respiratory equipment must be

sent to the hospital immediately. All involved must be mentioned in the incident report.

16.3   Incoming Compressed Gas Handling Procedures

16.3.1  Receiving personnel will leak check all compressed gas cylinders when received from the

vendor.

16.3.2  Leak Check Procedures

16.3.2.1  All incoming cylinders should be visually checked for signs of damage, cuts or

gouges in the metal, bent or damaged valves, burn marks, excess scale, rust or

corrosion, lack of decals or labels, or any other defect that may render the cylinder

unusable. These signs are indications of mishandling and possible problems.

16.3.2.2  All compressed gas cylinders received from the vendor are to receive a liquid snoop

gross check around the neck and dust cap of each cylinder.

16.3.2.3  If a leak is detected, follow the notification procedure in the "Response to a

Detected or Suspected Gas Leak" paragraph. DO NOT attempt a repair.

16.3.2.4  Respiratory Protection--SCBAs are to be readily available to personnel conducting

leak checks of toxic gases, and are to be used if a leak is detected.

16.3.2.5  If the leak is from the neck of the cylinder, immediately place the cylinder into the

leaking cylinder storage cabinet (wearing an SCBA). DO NOT make any further

attempt to stop the leak.

16.3.2.6  If the leak is a cross port leak, gently tighten the valve making sure that it was not

loosened during transport. If this does not stop the leak, attempt to tighten the dust

cap. If the leak continues, further leak stopping procedures will be determined by

the Safety Manager. Protective gear appropriate for each gas shall be worn.

16.3.2.7   If the leak cannot be controlled, place the cylinder in the leaking cylinder storage

cabinet. The Safety Manager will be responsible for notifying and obtaining

corrective action from the vendor.

16.3.2.8   If the leak is stopped, place the cylinder in the "used cylinder" shipping area for

return to the vendor.

16.3.2.9   All other handling procedures will be conducted under the direction of the Safety

Manager at his or her discretion.

16.3.2.10  Specific leak handling procedures involving inert gases will be determined on a

case-by-case basis by the Safety Manager.

16.3.2.11  Never place a leaking cylinder of toxic gas outside. The leaking cylinder should go

into the leaking cylinder gas cabinet. The Safety Manager will be responsible

for fulfilling all state and federal reporting requirements relating to the release of a

regulated gas to the environment.

SECTION 17: CUTTING, WELDING, AND BRAZING HOT WORK

NOTE: This major paragraph provides safe operating practices and requirements for LD

employees and all outside contractors while cutting, welding, or brazing to minimize accidents associated with these procedures.

NOTE: Deviations from the requirements of this major paragraph are not permitted unless area documents or local, state, or federal regulations are more stringent.

17.1    General Guidelines

17.1.1   No welding or other hot work can take place in areas where sprinklers are out of service.

17.1.2   The Safety Manager is responsible for ensuring that all outside contractors performing

welding of hot work in LD facilities are aware of and abide by the safe work

practices outlined in section 17.2.

17.2     Precautions for Hot Work

17.2.1   The supervisor of the area where hot work will occur must examine the location and

ensure the following conditions are met:

17.2.1.1  In areas where sprinklers are provided, they must be in service and remain in

service until the work is complete.

17.2.1.2  Work must be confined to the area or the equipment needing hot work.

17.2.1.3  The work area must be well ventilated.

17.2.1.4  Floors and surrounding area must be swept clean and wet down.

17.2.1.5  Ample extinguishers, water pails, and water hose lines long enough to cover the

work area must be provided in the work area.

17.3.2    The Safety Manager shall advise employees or contractors doing hot work where the hose

lines, fire extinguishers, fire blankets, and other safety equipment are located and how to

use each.

17.3.3    If the work involves cutting, welding, brazing, or other spark producing equipment, these

additional safeguards must be met:

17.3.3.1  All combustibles must be 30 to 40 feet from the operation. Those that cannot be

located 40 feet away must be protected by flameproof covers, or metal shields.

17.3.3.2  All floor and wall openings within 40 feet of the work area must be covered tightly.

17.3.3.3  A responsible person must be assigned to watch for dangerous sparks in the

area and in floors above and below, if applicable.

17.3.3.4  The watch must extend through the work period and at least 30 minutes after

work has been completed and during lunch and rest periods.

17.3.3.5  The person assigned to the fire watch must know where hose lines, fire

extinguishers, and fire blankets are kept and how to use each.

17.3.3.6  The person assigned to the fire watch must also know the emergency phone

number (9911) and the location of the nearest telephone.

SECTION 18: GENERAL SAFETY ORGANIZATIONS

18.1   Policy:  Larson Davis will provide a safe environment and healthful working

conditions in all facilities. The prevention of occupational injuries and illnesses is of such

consequence that it must be given precedence over operating productivity. To this end, it is

corporate policy that an effective program of occupational safety and health be the direct

responsibility of each member of management. This requires the development of safety awareness

in all LD employees. The Safety Committee provides an opportunity for the employees of

LD to work with the company to ensure that all safety practices and regulations are being

strictly followed. The Safety Committee is important to LD and is mandated by law.

18.1.2     The safety and health program includes the following:

18.1.2.1  Training all employees in sound safety and health practices.

18.1.2.2  Conducting a program of safety and health inspections to eliminate unsafe

working conditions and work practices, to control health hazards, to fully comply

with published safety standards for every job.

18.1.2.3  Providing all required and necessary personal protective equipment, as well as

the training for its use and care, as necessary.

18.1.2.4  Developing and enforcing safety and health standards and motivating all

employees to cooperate with those standards as a condition of employment.

18.1.2.5  Holding periodic Safety Committee meetings and orientations with employees to

discuss safety concerns.

18.1.2.6  Investigating every accident to determine its cause and taking appropriate action

so that the likelihood of its reoccurrence is minimized.

18.1.3     The assignment of responsibilities shall follow the guidelines below:

18.1.3.1  LD management is responsible for leadership in the safety and health

program, for its effectiveness, and for providing all safeguards necessary to

ensure safe working conditions. Management is responsible for developing the

proper attitudes toward safety and insuring that all operations are performed

with the highest regard for the safety and health of personnel involved.

18.1.3.2  The Safety Manager is responsible for establishing the procedures and

practices necessary to achieve an effective safety and health program.

18.1.3.3  The Safety Manager implements these safety and health procedures and

performs periodic audits/inspections to verify a safe and healthful work place.

Coordination across all operational lines is a necessity.

18.1.3.4  Employees are responsible for complying with all provisions of the safety and

health program, wearing the prescribed personal protective equipment provided

for each assignment, obeying all corporate safety standards, and exercising

good safety practices in the performance of their jobs.

18.1.4      LD Safety Committee member responsibilities are:

18.1.4.1  Attend all committee meetings.

18.1.4.2  Communicate safety observations in their own work areas and throughout the

facility in the course of the normal workday and correct hazards that can easily

be fixed.

18.1.4.3  Communicate safety problems to the Safety Committee they have observed and

others have brought to their attention.

18.1.4.4  Provide safety information and problem resolution feedback to all employees.

18.1.4.5  Be an example to others by performing their work safely.

18.1.4.6  Participate in and support the current Safety Committee programs for

employees, such as safety training courses and practical instruction.

18.1.4.7  Terms of membership shall be one year, or as determined by the committee.

Members wishing to remain in service longer are usually allowed to do so,

although it is a good idea to stagger terms of service to provide continuity.

18.2     Safety Audit Procedures.

18.2.1   In performing a safety audit or inspection, each committee member should be aware of the

following guidelines:

18.2.1.1  Audits shall be performed by the Safety Committee quarterly and should

be completed at least one week prior to the scheduled safety meeting to allow

time for inspection notes to be organized and to prepare for discussion.

18.2.1.2  A  LD “Safety Inspection” worksheet, (Form PE2016-4), shall be used

for all audits on site. As the committee performs its audit, one member shall act

as scribe and note violations and concerns, with at least two committee

members needed for an inspection.

18.2.1.3  Items found to be unsafe that can be taken care of on the spot should be

resolved by the committee members. Items of greater magnatude should be

brought to the attention of the supervisor of the area and a timeframe for

completion negotiated and noted on the inspection form.

18.2.2      There are basically four types of problems that may be observed by or brought to the

attention of the safety committee. They may range from major to minor in level of hazard:

    - -Hazardous physical conditions, such as blocked exits, trip hazards, exposed wires various equipment problems, etc.

    - -Safety equipment usage, such as the use or misuse of personal protective equipment.

    - -Environmental, such as temperature variations, odors and fumes, lighting, and other factors relating to employee perceptions of health and comfort.

    - -Organizational activities, such as evacuation drills, equipment training, etc.

18.2.3      An emergency situation is one in which imminent danger to employees is present, such

as a fire, large chemical spill, toxic gas leak, etc. In such cases the Safety Manager must

be notified immediately.

18.2.4      A serious situation is one where the possibility of injury to an employee may be

significant, such as non-use of required safety equipment, unguarded mechanical

equipment, exposed electrical connections, slippery walk surfaces, blocked exits,

trip hazards, etc. In such cases the Safety Committee should:

Immediately contact the responsible supervisor of the equipment and/or person involved

in the hazard and tactfully point out the problem, requesting that corrections be made.

In a situation where employee function in a hazardous situation will result in injury, the

employee should be warned immediately and the supervisor notified as stated above.

Alert the Safety Manager for followup action. The Safety Manager has the responsibility to

approach the proper management levels to insure corrective action takes place. If the

corrective action requires a major work order or change in LD safety policies,

the Safety Manager should then inform the facility General Manager, who will then review

the situation, and either immediately terminate the hazardous operation or determine

suitability as an item for discussion at the next LD Safety Council Meeting.

18.2.5     A non-serious hazard, such as temperature control, burned out light globes, and other

lesser problems, may usually wait to be discussed at the safety committee meeting. At

that time the Safety Manager may delegate a particular member of the committee to

address and resolve the problem.

18.3   Larson Davis Safety Council

18.3.1     Duties: The LD Safety Council shall act as a policy making body in promoting a

safe healthful environment for all LD employees. The council consists of the

Safety Manager, the Logistics Manager, the Production Manager,the Quality Assurance

Manager and the General Manager.

The council will evaluate and implement safety and health policies.  It shall provide top

management support in correcting action items and will establish an effective means of

communicating safety inniatives to all employees, including promotional campaigns, and

progress status reports.  In an effort to promote corporate safety goals, the council will:

18.3.1.1  Set facility safety goals and policies, prioritize safety programs, and provide

direction to safety efforts within the LD site organization.

18.3.1.2  When necessary, render facility decisions to correct unsafe conditions with

regards to:

- Funding

- Responsibility

- Priority

- Policy

18.3.1.3  Address safety concerns and problems submitted by individual workers and/or

the safety committee.

18.3.1.4  Provide direction, guidance, and feedback to the facility Safety Committee on

their recommendations for safety improvements.

18.3.1.5  Review LD accident/injury rates, worker's compensation cases, safety

citations, etc.

18.3.1.6  Ensure all areas of the company are represented on the Safety Committee.

18.3.1.7  Ensure that an emergency response team (ERT) exists at the LD site.

18.3.1.8  Special meetings requiring priority action may be called at the discretion of the

General Manager, otherwise meetings will be held on a regular basis.

SECTION 19: SAFETY AND HAZARDOUS WASTE TRAINING

19.1     Basic Chemical Safety.

19.1.1   Basic Chemical Safety training is given to all production area employees as part of their

new hire orientation, and is updated annually, if their job entails the use of chemicals.

19.1.2   This training is intended for employees working with chemicals in fully controlled

environments.

19.1.2.1  Controlled environments are work areas in which the process chemicals are

enclosed, ventilated, handled in small amounts in squeeze bottles or otherwise

contained, i.e., ventilated acid sinks, solvent sinks or vats, and the intermittent

use of spot cleaning solvents in normally ventilated areas.

19.1.3   The Safety Manager or safety trainer will administer Basic Chemical Safety training and

maintain training records on file.

19.2      Basic Safety Orientation.

19.2.1   Part of each employee’s job training includes being certified in the company’s safety

procedures, including Material Safety Data Sheet (MSDS) training.

19.2.2   This training is given by the Safety Manager or safety trainer to new employees as part of

their new-hire orientation during the first week of employment.

19.2.2.1  Records are kept in the Safety Manager’s file.

19.3      Chemical Handler Training.

19.3.1   This training is intended for workers whose jobs required them to work with hazardous

chemicals in situations where exposures may not be fully controlled.

19.3.1.1  Examples of uncontrolled situations include equipment repair, chemical

transfers outside ventilation hoods, and chemical spill clean-up.

19.3.1.2  Chemical handler training will be required before a designated worker is allowed

to work without direct supervision.

19.3.1.3  Never handle chemicals when working alone in an area without supervison or

without an immediate backup person.

19.3.1.4  Annual recertification training will be required.

19.4.1    To be trained as a chemical handler, an individual must complete the following:

Basic Chemical Safety

Hazardous Chemicals

Spill Control and Clean-Up

Respiratory Protection

Waste Disposal Procedures

19.5    Waste Disposal Training.

19.5.1   This certification is required of all persons who "process" material which

could be considered hazardous waste by regulation.

19.5.1.1  "Processing" means using material in various work functions, consolidating used

material, and packaging the materials for shipment after use. This may entail

applying required DOT and RCRA labels to the storage containers, and keeping

records of wastes generated by the facility.

19.5.2   Waste disposal training is required before a worker is allowed to process waste without

direct supervision.

19.5.2.1  Annual training will be required to maintain certification.

19.5.3   To be waste disposal certified, an individual must complete the following classes:

- Chemical Handler

- Hazardous Waste Packing, Labeling and Records

- Emergency Procedures

19.5.4  These courses will be coordinated by the Safety Manager.

SECTION 20: CHEMICAL TERM DEFINITIONS

20.1       Acid: A corrosive with a pH less than 7. Strong acids have a pH less than 2 (see corrosives).

20.2       Base: A corrosive of pH greater than 7. DANGEROUS WHEN WET. Also known as Aklaline.

20.3      Corrosive: Capable of dissolving organic tissue. Destroys by chemical action. If pH value is less

than 2, it is an acid; if greater than 12.5, it is a base.

20.4      Flammable: Capable of burning. Any chemical with a flash point below 100 Degrees Fahrenheit is

flammable.

20.5      Flash Point: The temperature at which a liquid gives off enough vapor to be ignited.

20.6      Oxidizer: Produces or gives off oxygen. An oxidizer increase the flammability of other materials.

20.7       p.p.m.: Parts per million is a unit of measurement used in measuring levels of toxic materials in

air or water.

20.8      T.L.V.: Threshold Limit Value. T.L.V. is the legal limit of exposure to employees of chemical

fumes, dust or mist. This figure refers to an average exposure over an eight hour period.

20.9      Toxic: Poisonous—produces changes to the body that may cause injury or illness.

SECTION 21: PERSONAL PROTECTIVE EQUIPMENT DEFINITIONS

21.1      Definitions:

20.1.1   Air Purifying Respirator (APR): A respiratory device that removes contaminants from air.

20.1.2   Chemical Goggles: Goggles with ventilation ports hooded in a manner that will allow air to

circulate around the eyes and prevent splashed liquids from entering the ports.

20.1.3   Demand Mode: A self-contained breathing apparatus (SCBA) in which air is supplied to

the facepiece only when the user inhales. Since a negative pressure is created inside the

facepiece when the user inhales, a SCBA used in the demand mode is no more efficient

than an air purifying respirator with the same facepiece.

20.1.4   Full-Facepiece Respirator: An APR that uses an appropriate type of cartridge (organic

vapor, acid gas, or HEPA) to purify the air in an atmosphere in which the contaminant

does not exceed 50 times the OSHA permissible exposure limit (PEL).

20.1.5   Full-Face Shields (Clear, Tinted): Shields used to protect the face and eyes against

impact from flying objects and corrosive liquids. Clear face shields are used to protect

against light impact and corrosive liquids. Tinted face shields are used to protect against

ultraviolet light, light impact, and corrosive liquids.

20.1.6   Half-Mask Respirator: An APR that uses an appropriate type of cartridge (organic vapor,

acid gas, or HEPA) to purify the air in an atmosphere in which the contaminant does not

exceed 10 times the OSHA PEL. Half-mask respirators are NOT approved for use with

eye irritants.

20.1.7   Hearing Conservation Program: A program that protects and promotes good safety

practices. Engineering-controls will be installed to reduce noise exposure to safe levels.

However, certain jobs that are infrequently done by technicians, maintenance workers,

and others may require the use of hearing protection.

20.1.8    Laser Safety Goggles: Goggles specifically designed to protect against radiation from

the laser system used when Engineering and procedural controls are inadequate to

eliminate potentially harmful exposure.

20.1.9    Pressure Demand Mode: An SCBA mode in which a slight amount of air passes

continuously through the facepiece maintaining a positive pressure inside the facepiece.

Any facepiece leakage is outwards, and the OSHA permissible exposure limit of up to

10,000 times can be maintained.

20.1.10  Protective Gloves: Gloves used as protection from solvents (neoprene) or from

corrosives (orange latex). Gloves of proper type and material are provided for hand

protection in work involving the handling of sharp or rough objects, corrosive liquids,

flammable liquids, and materials or equipment under abnormal conditions or at

temperature extremes. The use of asbestos fiber gloves is prohibited at LD.

20.1.11  Respiratory Protection Coordinator: The Safety Manager, who has been trained in the

use and limitations of respiratory protection equipment and who is responsible for

administering the respiratory protection program.

20.1.12  Safety Glasses with Wrap-Around Side Shields (Clear or Tinted): Glasses made of an

impact resistant plastic that meet current ANSI standards. The side shields prevent

foreign bodies from entering from top or side. These safety glasses are standard for

fabrication and other production areas and are also used by visitors.

20.1.13  Self-Contained Breathing Apparatus (SCBA): An open-circuit respiratory device

equipped with a high pressure compressed air tank. The exhaled air is exhausted to

the atmosphere. The compressed air from the tank is delivered to a two-stage

regulator that reduces the air pressure and provides air to the facepiece on demand.

20.1.14  Steel-Toe/Steel-Shank Chemical-Resistant Safety Shoes: Steel-toe chemical-resistant

safety shoes are similar in appearance to ordinary safety shoes. They differ in that they

are made of leather with a one piece tongue and a chemical (acid) resistant sole.

These shoes are intended to protect against falling objects and chemical contact.

20.1.15  Welding Goggles: Goggles worn over safety glasses or ordinary glasses. They are

designed in a manner to permit easy changing of the lenses so that the shade of the

lens may be changed to meet the requirements of the particular type of welding.

20.1.16  Welding Hood: A hood made of strong, non-combustible, opaque materials shaped to

protect the entire face and ears from flying sparks, molten metals, and injurious rays of

light. The hood is equipped with a colored lens to view the work.

SECTION 22: PROTECTIVE EQUIPMENT GENERAL INSTRUCTIONS

22.1    Safety Manager Responsibilities.

22.1.1   Evaluate work conditions that expose employees to hazardous conditions, and

review and approve acceptable protective equipment to be stocked.

22.1.2   Ensure that each applicable supervisor and employee receives training in the use

of their protective equipment.

22.1.3   Perform periodic inspections of proper protective equipment usage and storage in

applicable areas.

22.2    Supervisor Responsibilities.

22.2.1   Ensure that no employee begins work requiring personal protective equipment until

that employee has been trained by the Safety Manager or Safety Trainer in the types of

equipment necessary for the job and the proper use of the equipment.

22.2.2   Ensure that each employee who uses protective equipment complies with the

requirements of this specification. This includes ensuring that each employee wears the

required protective equipment at all times.

22.2.3   Maintain required-protective equipment for employee needs.

NOTE: Supervisors in areas requiring the use of protective equipment are

responsible for the cleanliness and serviceability of the equipment.

22.3     Employee Responsibilities.

22.3.1   Do NOT use safety equipment as a substitute for adequate operator procedures or good

personal judgement.

22.3.2   Wear required protective equipment only for situations or jobs for which protective

equipment is required, such as back braces when lifting, knee pads when kneeling, etc.

22.3.3   Report any missing, damaged, or otherwise unacceptable protective equipment to your

supervisor.

22.3.4   Become trained in the use of personal protective equipment.

SECTION 23: EYEWASH FOUNTAINS AND EYEWASH BOTTLES

23.1      Safety Manager responsibilities.

23.1.1   Provide prior approval of the installation or relocation of each eyewash fountain or bottles.

23.1.2   Ensure that emergency eyewash fountains are provided in each area where an employee's

eyes or body may be exposed to a splash hazard from any injurious substance.

23.1.3    Ensure that eyewash bottles are installed in areas where installation of an emergency eye

fountain is not feasible.

23.1.4    If an emergency eyewash fountain is temporarily out-of-order, ensure that

auxiliary equipment is provided.

23.1.5    Equip each electrical box, each switch, and all equipment on which water might be

splashed with a ground fault interrupt device.

23.1.6    In each area where an emergency eyewash fountain, or eyewash bottle is

located, post a sign in a conspicuous place with either one of the following

messages:

  - EMERGENCY EYEWASH FOUNTAIN

  - EYEWASH BOTTLE

23.1.7    Post operating instructions near each emergency eyewash fountain, and eyewash bottle

where the employee may easily read the instructions and learn how to use the facilities.

23.1.8    Ensure that an emergency eyewash fountain is located within 25 feet walking distance or

15 second travel time of each potential hazard, depending upon the nature of the hazard.

23.1.9    Store each eyewash bottle in a labeled eyewash station and ensure that each

eyewash bottle is equipped with a cover.

23.1.10  Ensure that each valve used on an emergency eyewash fountain water supply line meets

the following requirements:

    - Valve designed to indicate whether it is open or closed

    - Valve locked in the open position

    - Valve clearly labeled according to the type of service it controls

    - The water supply to eyewash fountains do NOT go through an intermediate valve that may be closed when other equipment is being serviced.

23.1.11  Ensure that all water supplies and drains servicing emergency eyewash fountains are

properly installed and connected to meet required flow standards.

23.1.12  Ensure that each eyewash fountain has a low pressure water supply with a discharge

rate of at least 2.5 gallons per minute. If possible, areate the water stream for each

eyewash fountain.

23.1.13  Ensure that each eyewash fountain has two streams of water that are high enough to

flush both the face and the eyes.

23.1.14  Perform a quarterly test on each eyewash fountain, using

Form PE2016-5 “Safety Eyewash Inspection” for the following requirements:

    - Low water level (less than 3/4 full) for eyewash bottles

    - Growth or discoloration on the inside surface of the bottle

    - Sediment at the bottom of the bottle

    - Eyewash station operative with water holes not plugged

23.2    LD Employee Responsibilities.

23.2.1    Ensure that each eyewash bottle is inspected weekly.

23.2.2    Bring any of the following deficiencies to the attention of the Safety Manager:

    - Low water level (less than 3/4 full) for eyewash bottles

    - Growth or discoloration on the inside surface of the bottle

    - Sediment at the bottom of the bottle

23.2.3   At all times, maintain a clear pathway and an unobstructed access to each emergency

eyewash fountain, and eyewash bottle.

23.3    Safety Eyewash Fountain Use.

NOTE: The purpose of the wash stations is to dilute and flush away toxic materials before

serious effects occur.

23.3.1    At the eyewash station, flush the affected area with water continually for 20

minutes.

23.3.2    During the irrigation of the eyes, hold the eyelids open to ensure that the entire

surface of the eyes and eyelids are thoroughly flushed.

23.3.3    After rinsing, IMMEDIATELY contact the Safety Manager or the site Emergency Medical

Technican.

23.5    Eyewash Bottles

23.5.1   Only remove the eyewash bottle from the eyewash station when the bottle is being used

or inspected. The bottle is not to be any place other than the eyewash bottle station.

23.5.4    If an injurious substance is splashed into an eye and an eyewash fountain is not located in

the immediate area, do the following:

    - Break the seal and open the eyewash bottle.

    - Flush the eye by tipping the bottle to the eye and inverting the bottle. Do NOT squeeze the bottle.

    - Hold the eye open to dilute and flush out the contaminant.

    - Proceed to an eyewash fountain and continue rinsing out the eye for 15 minutes.

    - Seek medical attention by contacting the Safety Manager or site Emergency Medical Technician.

SECTION 24: CHEMICAL APRONS AND SUITS

24.1      Wear a chemical apron or a suit when working with any chemical, except for paints and squirt

bottles of 1000ml or less, unless otherwise directed for a given material.

24.1.1   Ensure that your chemical apron or suit is clearly marked so that the outside (the

potentially contaminated side) will not accidently be worn next to the body.

24.1.2   After each use and prior to removal, wipe the apron or suit with a wet towel (water).

24.1.3   Before and after each use, ensure that your apron or suit is free of tears or leakage.

24.1.4   In the event of spills, use care to flush away all contaminants before removing the

chemical apron or suit.

SECTION 25: GLOVES

25.1      Gloves are to be worn when working with ANY chemical, except for paints and small quantities of

chemicals in squirt bottles or beakers of 500ml or less, unless directed otherwise.

25.2       Do NOT wear gloves when working with machinery that presents a hazard of gloves being caught

by rotating or moving gears, chains, or shafts.

25.3       Do NOT wear jewelry that may puncture protective gloves.

25.4       Glove Liners

25.4.1  Wear glove liners with the required gloves for added protection against acids and other

hazardous chemicals.

25.4.2  Do NOT wear glove liners as a substitute for the required gloves.

25.5       Glove Inspection Test

25.5.1    Perform this test before each use of a new pair of protective gloves.

25.5.2    Open the glove completely.

25.5.3    Hold the glove by the open-end firmly with both hands.

25.5.4    Quickly close the end of the glove until the glove inflates.

25.5.5    Visually check for possible leakage.

25.5.6    If pinholes are not found, put on the gloves. If pinholes are found, discard and obtain

another pair.

25.5.7    Roll down the gloves at least 1-2 inches.

NOTE: This forms a cuff that prevents hazardous chemicals from coming in contact with

the arms. If the gloves are 15 inches or greater in length and the hands are not

frequently raised above elbow height, they do not need to be cuffed.

25.6      Glove Cleaning

25.6.1    After working with chemicals, rinse and remove the gloves before handling equipment.

25.6.2    Rinse gloves in a water rinse for 10 seconds.

25.6.3    If the gloves become ripped, torn, or dirty, replace them.

25.6.4    Replace latex gloves daily.

25.6.5    Dispose of contaminated gloves in a trash receptacle only after rinsing them off.

SECTION 26: EYE PROTECTION

26.1      Safety Manager Responsibilities.

26.1.1    Evaluate work conditions, and determine the areas where an eye hazard exists to

employees.

26.1.2    Designate areas that are an eye hazard. For each mandatory eye protection area, post a

sign "EYE PROTECTION REQUIRED."

26.1.3    Review and approve acceptable eye protection to be stocked.

26.1.4    Perform periodic inspections of proper eye protection usage and of storage in mandatory

eye protection and other required areas.

26.1.5    Provide eye protection for visitors in areas that require eye protection, and ensure an

adequate supply of safety glasses exists in these areas.

26.2      Employee Responsibilities.

26.2.1    Do NOT wear contact lenses if any of the following operations are performed:

    - Exposure to corrosive or irritant liquids, gases, or vapors

    - Exposure to dusts, sandblasting, or fiberglass application

    - Exposure to flammable liquids and solvents

    - Wearing self-contained breathing apparatus

    - Welding operations

26.2.2   Wear the required eye protection equipment.

NOTE: Wearing of eye protection equipment in mandatory eye protection areas is a

condition of employment. The employee is responsible for conscientious use of eye

protection equipment.

26.2.3   Eye protection will be required when any of the following operations are performed:

    - Working with any amount of a chemical

    - Cutting component leads on printed circuit boards

    - Soldering

    - Grinding

    - Drilling

    - Shearing or cutting

    - Buffing

    - Machining

    - Using air or electrically driven hand tools

    - Working with or near pressurized systems

    - Cutting and sawing

    - Welding or brazing (Note: special safety-tinted glasses specific for these tasks are required).

    - Working around electrical arcs

    - When de-energizing or energizing high voltage switches

    - Working with equipment that emits high intensity

26.2.4   If your vision is limited to one eye, you must wear safety glasses at all times.

26.2.5   When safety glasses are the minimum eye protection equipment required for an area

and you wear prescription glasses, wear any of the following glasses:

    - Prescription safety glasses with side shields

    - Chemical goggles worn over prescription glasses

    - Full face shield

    - Safety glasses over prescription glasses

SECTION 27:  FOOT PROTECTION

27.1      Safety Manager Responsibilities.

27.1.1    Ensure that needs and requirements of adequate foot protection are identified and that

proper foot protection guidelines are provided.

27.2      Employee Responsibilities.

27.2.1   Wear the required foot protection in mandatory foot protection areas.

Note: The Safety Manager and supervisor may specify the correct footware appropriate

for a work area. Protective footware must be worn in mandatory foot protection areas.

27.3      Areas requiring steel toe shoes:

- Moving equipment with a weight of 100 pounds or more.

- Locations where there is a possibility of heavy material falling on the feet.

27.4       Areas requiring closed toe (no sandals), leather (not canvas) shoes:

- Areas where chemcials are used, except for amounts less than 500ml in squirt bottles or paints.

- Any warehouse, storage, or material stores/shipping areas.

SECTION 28: HEAD PROTECTION

28.1      Safety Helmets (Employees and Visitors).

28.1.1    If a hazard of objects falling from above or a danger of bumping the head against

overhead structures, moving equipment, or swinging loads of material exists, wear a

safety helmet.

28.1.2    Wear safety helmets while in the following areas or under the following conditions:

- When overhead operations are in progress, such as forklift use (lookout included)

- While in construction areas

- When designated by the Safety Manager

28.1.3    If you have a skull injury or skull plates, wear safety helmets at all times, except when in

an office area or the break rooms.

28.1.4    For electricians and electronic technicians and if safety helmets are required, wear an

approved, non-conductive electrical worker's safety helmet.

28.2       Hair (Employees and Visitors)

28.2.1    While working with or in the vicinity of moving machinery, wear hair nets, hair ties,

disposable caps, or other hair protection equipment to cover and hold the hair.

SECTION 29:  HEARING CONSERVATION PROGRAM

29.1    Safety Manager Responsibilities.

29.1.1    Monitor all areas potentially greater than 80 dBA with calibrated, sound measuring

equipment on a regularly scheduled basis and all noise exposure areas any time when

conditions change.

29.1.2    Maintain data and all reports on noise measurements conducted within the facility.

29.1.3    Recommend audiograms for personnel in specific noise exposure areas.

29.1.4    Provide hearing conservation training for affected employees.

29.1.5    Select appropriate hearing protection devices.

29.1.6    Investigate noise controls for equipment that produce noise levels in excess of 85 dBA.

29.1.7    Assess potential noise output of proposed new equipment purchases.

29.1.7.1   Ensure that new equipment will not produce noise greater than 80 dBA at 3 feet.

29.1.8    Identify new employees and employees that transfer into work areas where noise levels

exceed 85 dBA, and proceed as follows:

Conduct Hearing Conservation Program training for employees receiving audiograms.

Coordinate employee audiograms as specified in the OSHA Standard Reference, 29 CFR

1910.95(c).

Ensure that audiograms are reviewed by a qualified physician or certified licensed

audiologist.

Ensure that employees are informed of the audiogram results, and coordinate referrals to

specialists as needed.

Ensure that the results of all audiograms are maintained in the employee's medical file.

Ensure that employees who have been working in areas where the noise level exceeds

85 dBA receive an audiogram upon termination of employment from PCB.

Instruct employees about the proper use and fitting of foam ear plugs and consult in the

selection and use of other ear plugs as specified in the OSHA Standard Reference, 29

CFR 1910.95(c).

29.2      Employee Responsibilities

29.2.1    If hearing protection devices are required, only wear equipment that is in good condition

according to instructions provided by the Safety Manager.

SECTION 30: RESPIRATORY PROTECTION

NOTE: Employees found in violation of the requirements of this major paragraph may have their respirator certification revoked.

30.1      Respiratory Protection Program

30.1.1    The following items will be included in the LD Respiratory Protection Training

course:

    - Appropriate cartridges and filter use

    - Cleaning and sanitizing

    - Fit testing Inspection and maintenance

    - Proper selection

    - Storing

    - Use limitations

    - User medical limitations

30.1.2     Maintain a record of each employee who receives respiratory protection training, fit

testing, and medical examinations, if any.

30.1.3     Ensure that the appropriate types and quantities of respirators, filters, cartridges, and

canisters are stocked.

30.1.4     Ensure that each employee who is required to use respiratory protection receives

training and semi-annual fit testing.

30.1.5     Ensure that each employee who is required to use respiratory protection has filled out

the “Medical History Questionaire” (Form F)and has the physical ability to use a

respirator.

NOTE: An employee who has not successfully completed the LD Respiratory

Protection Training course and has not been certified to use a respirator by the Safety

Manager, is not allowed to use a respirator under any circumstances.

30.1.6     Ensure that the intended use of each respirator used at the LD site has been

approved by the National Institute for Occupational Safety and Health (NIOSH) and by

the Mine Safety and Health Administration (MSHA).

30.1.7     Control the use of respiratory protection.

30.1.8     Evaluate work conditions that expose employees to hazardous airborne substances,

and determine when appropriate respiratory protection is required.

30.2      Employee Responsibilities.

30.2.1    Complete a “Medical History Questionaire”, (Form PE2016-6).

30.2.1.1    The Safety Manager will evaluate the type of work performed and the medical

condition of the user as specified in the OSHA Standard Reference, 29 CFR

1910.134.

30.2.2    The LD employee will wear respiratory protection when conditions warrant.

30.2.3    The employee will agree to properly store and maintain the respirator issued to them.

30.2.4    The employee will be clean shaven. Mustaches are allowed but beards are not if

respiratory protection is required in the course of an employee’s job functions. This

standard is a condition of employment.

NOTE: For a respirator to provide the protection for which it is designed, the respirator

facepiece must fit the face of the wearer properly. Facial hair inhibits the proper fit.

30.3      Positive Pressure Test

30.3.1    Hold the exhalation valve closed with the palm of your hand and exhale lightly into the

facepiece.

NOTE: The facepiece fits properly if a slight positive pressure is built up inside the

respirator and no outward leakage occurs along the face seal.

30.4     Negative Pressure Test

30.4.1    Cover the air inlet of each filter or cartridge with palm of your hand or squeeze the

breathing tube to prevent air from passing through the tube.

30.4.2    Inhale lightly until the facepiece collapses slightly.

30.4.3    Hold your breath for 10 seconds.

NOTE: The facepiece fits properly if it remains collapsed for 10 seconds and no inward

leakage occurs.

30.4.4    The employee must pass the isoamyl acetate vapor test as specified by 29 CFR 1910.

1001,  Appendix CI, to be certified to wear a respirator..

30.4.5    If the employee passes all tests, they will be authorized to wear a respirator for one year.

30.5      In Oxygen deficient atmospheres, less than 19.5% Oxygen as specified by OSHA, or with

unknown contaminants of or unknown concentrations of contaminants, a self contained beathing

apparatus (SCBA) shall be used.

30.6      With Oxygen atmospheres greater than 19.5% Oxygen, but with eye irritants present, a full face

air purifying respirator shall be used. If acid gases are present, acid gas cartridges shall be used.

30.7      With Oxygen atmospheres greater than 19.5% Oxygen with organic chemicals present, a half

mask, air purifying respirator shall be used.

30.8      An air supplied or air purifying respirator shall be chosen for use depending on the expected level

and the properties of each contaminant and the duration of the exposure to the contaminant.

30.9      Under no circumstances shall a respirator be shared by employees. Respirators are fit-specific for

each employee.

30.10    Respirators shall be stored in enclosed containers. Plastic bags or resealable plastic

containers are acceptable storage locations.

30.11    Pre-use air purifying respirator inspection by the employee.

30.11.1   Check the cleanliness of each respirator.

30.11.2    Inspect the following parts:

    - Cartridges

    - Facepiece

    - Straps

    - Valves

30.11.3    If any part is dirty, damaged, or worn, or if any part is missing, proceed as follows:

30.11.3.1   Immediately return the respirator to the Safety Manager.

30.11.3.2   Obtain a new respirator, or if replacement parts are not available, replace

the respirator.

30.11.3.3   Do not interchange parts between different brands or types of respirators.

30.12    Pre-use SCBA inspection by the employee:

30.12.1     Inspect the following parts parts of SCBA gear:

    - Air tank level

    - Facepiece

    - Hoses and connections

    - Regulator

    - Straps

    - Valve covers and gaskets

    - Valves

    - Warning devices

30.12.2     If any part is dirty, damaged, or worn, proceed as specified in section 30.11.3.

30.13    Respirator usage guidelines for the employee.

NOTE: Each respirator user's department may stockpile appropriate filters, cartridges, and

canisters, but not respirators, due to their fit-specific nature.

30.13.1     Supervisor responsibilities are to ensure that each employee who wears a respirator:

-Is physically fit to wear a respirator

-Is free of facial hair that may interfere with the face-to-respirator seal

-Is fitted properly and certified by the Safety Manager to wear the respirator

30.13.2    Employee responsibilities are to ensure that:

-They wear a respirator only after receiving proper certification and training.

-They do not wear contact lenses in any atmosphere that requires a respirator.

-They do NOT have facial hair that interferes with the face-to-respirator seal or that

may interfere with a valve.

NOTE: An employee with facial hair will have their respirator certification

revoked.  Skull caps and facial scars can also prevent a good fit.

30.14        Before entering a contaminated atmosphere, do the following:

30.14.1   Perform the positive pressure test as specified in paragraph 30.3.

30.14.2   Perform the negative pressure test as specified in paragraph 30.4.

30.14.3   Adjust the respirator as needed for extended comfort.

30.15        Post use inspection by the employee.

30.15.1   Perform the procedures specified in paragraph 30.11.

30.15.2   After each use, inspect the storage conditions to protect strap and integrity of the

respirator.

30.16        Personal respirator cleaning by the employee.

30.16.1   At least once every three months, clean and sanitize the respirator.

NOTE: Cleaning and disinfecting the respirator once every three months or

after each use is sufficient for most circumstances.

30.16.2    Do the following to clean and sanitize the respirator:

30.16.2.1   Remove each filter, strap, and air valve.

30.16.2.2   Wash the respirator in a mild soap and water solution using a soft brush.

30.16.2.3   Rinse the respirator thoroughly with clean,  warm water.

30.16.2.4   Mix a 100 ppm chlorine solution by adding 4 ml of bleach to 1 liter of

water and immerse the respirator in the chlorine solution for 2 minutes.

30.16.2.5   Thoroughly rinse the respirator again with clean, warm water.

30.16.2.5   Allow the respirator to dry before storing it.

30.17     Monthly SCBA audits.

30.17.1        Perform a monthly inspection of each SCBA kept for emergency use.

30.17.2        Test regulators and warning devices to ensure that they are functioning properly.

30.17.3        Check the facepiece, hoses, connections, straps, valve covers, and gaskets for

signs of deterioration.

30.17.4        Review the faceplate storage conditions to ensure the shape and integrity of the

respirator is maintained.

30.17.5        Replace or repair damaged regulators.

30.17.5.1   Send each regulator to the manufacturer to be certified every 3 years.

30.17.6       Fill or replace any cylinder that is not fully charged.

30.17.6.1    Send cylinders to the vendor to be hydro-tested, type “C” every 3 years

and type "S" (steel) every 5 years.

SECTION 31: PROTECTIVE EQUIPMENT STORAGE

31.1         When not in use, clean and store personal protective equipment in specially designated storage

area convenient to the work area.

31.2         Do NOT hang personal protective equipment or clothing on doors, door knobs, electrical switch

boxes, or any other unauthorized location.

31.3         The storage area must be designed so that personal protective equipment or clothing cannot

become contaminated with hazardous substances during storage.

31.4    Respirator Storage.

NOTE:  Improperly stored respirators are subject to confiscation.

31.4.1   Store each respirator in a location or storage unit the will prevent dirt, chemicals,

sunlight, or other elements from causing the respirator to deteriorate. Storing the

respirator in a plastic bag will usually satisfy this requirement.

31.4.2   Store each respirator in a secure location that will prevent other employees from

tampering with the respirator.

31.4.3   Store each respirator so that the facepiece and valves rest in a normal position.

31.4.4   If the respirator is stored in a locker, keep it in a carton or carrying case.

31.4.5   Do NOT store the respirator in a tool box.

SECTION 32: ERGONOMICS

32.1     Posture at the work station.

32.1.1    Proper work height with relation to the worker is essential for the comfort and well being

of the employee. As awkward positions and postures are employee specific, it is up to the

employee to bring to the attention of the supervisor any posture related difficulties that

they may be having.

32.1.1.1  Seat and work table height adjustments are usually sufficient to improve

posture related problems.

32.1.1.2  Objects required at the work station should be placed within reach to

discourage leaning or bending of the body. Reaching above shoulder height

should be kept to a minimum.

32.2     Repetitive work conditions.

32.2.1     Employees should allow self pacing, with rest pauses if tired and utilize safety equipment,

such as lace-on leather or plastic wrist supports, to prevent cumulative trauma disorder

to their hands. A work station wrist height pad can be requested from the supervisor to

change the angle of the wrist with relation to the work and alleviate most discomfort.

32.2.2     Alternate hands or limbs if possible. Rotate tasks or job elements to be completed to

reduce the repetitiveness of any one activity.

32.2.3     Minimize the force, duration and frequency of work activities, and use hand tools in a

manner that keeps the wrist from bending and straightening.

32.3     Temperature considerations.

32.3.1    The work areas will be kept at an appropriate comfortable temperature, as conditions,

seasons, and facility equipment will allow.

32.3.1.1   Only those authorized are allowed to change thermostat settings. As no two

employees can agree to what is an appropriate temperature setting, space

heaters and small fans will be permitted in non-hazardous work areas, at the

discretion and approval of the Safety Manager.

32.4     Lighting of the workplace.

32.4.1   To prevent eye stain and ensure the safety of work areas, bring any burned out light

globes or tubes to the attention of your supervisor. Replacement should immediately occur

if the lack of light increases the possibility of trip hazards, head trauma, etc.

32.5     Lifting material at the job site.

32.5.1    Employees should use judgement as to their personal capability and the

suitability of whether a weight should be lifted. Back support belts will be available to

those employees that request their use. Under no circumstances shall an employee

attempt to lift more than 50 lbs. without help from another worker or the use of lifting

equipment, such as forklifts or hoists.

32.5.2    Employees who have previously injured their backs, have had hernias, are currently

pregnant or have some other physical condition that prevents them from safe lifting,

should bring this condition to the attention of their supervisor. EMPLOYEES WITH ANY

MEDICAL CONDITION THAT PREVENTS SAFE LIFTING ARE NOT TO ATTEMPT TO

LIFT ANY MATERIAL AT LD, NO EXCEPTIONS.

32.5.3   When lifting is undertaken, proper lifting technique is the only procedure authorized to be

used at LD:

    - The weight should never be above the shoulders in height.

    - Weight should be kept as close to the body as possible.

    - You should not bend at the waist, but bend the knees to pick up a weight.

    - Do not twist your body as the weight is lifted.

    - Use a back support and gloves if you think you need them to properly lift material.

    - No more than 50 lbs. is to be lifted without the aid of powered lifting equipment.

32.6       Computers and video display terminals.

32.6.1    To prevent cumulative trauma disorder, wrist height should be adjusted with a wrist pad

to prevent bending of the hand and wrist when working on a keyboard. These will be

available, if requested. Those doing extensive data entry or computer work can request

a specialized, ergonomically enhanced keyboard.

32.6.2    Monitor height should be appropriate to your seated height, and adjusted in angle to

prevent neck muscle strain and eye fatigue.

32.6.2.1  Eye strain can be reduced by taking intermitent breaks and utilizing a glare

controlling screen on your computer monitor. These are available on request.

32.7     Trip hazards in the workplace.

32.7.1  Aisles and hallways must be kept free from obstructions. These are fire exits and not

storage areas.

32.7.2  Electrical cords may be run across aisles or hallways, but only with the Safety Manager’s

approval. Cords must be covered with a rubber aisle cord cover, available from the Safety

Manager, or taped to the floor. Tape will only be used on a short term basis.

32.7.2.1  Covered electrical cords across hallways or aisles are only considered a short

term, interim event out of neccessity. This will never be considered a permanent

situation and facilities will be changed to accomidate any new electrical needs.

SECTION 33: HAND AND POWER TOOL SAFETY

33.1     General guidelines for power and hand tool safety.

NOTE: HAND OR POWER TOOLS ARE ONLY TO BE USED BY THOSE EMPLOYEES

TRAINED TO USE THEM. ONLY USE THE TOOLS REQUIRED IN YOUR JOB.

33.1.1   Electrical hazards can result from faulty electrical wiring within the tool itself or because of

contact of the tool with an electrical source.

33.1.1.1   Inspect the power tool prior to use and do not use it if wires are frayed,

exposed, or the tool doesn’t seem to be working properly. Bring the problem to

the immediate attention of your supervisor.

33.1.1.2   Use extreme care when working around electrical wires that may be hidden

and damaged by power tools you are using.

33.1.1.3    Proper grounding is essential for proper use of electrically powered tools.

Never bypass or cut off the grounding plug. The power circuit that the power

tool is on should be fused.

33.1.1.4   Pneumatic tools should have secure fittings  and supply lines in good shape to

prevent unexpected disconnection and whipping of supply lines.

33.1.2    Mechanical hazards are usually prevented by safety guards intrinsic to the power tool. If

these are present, they are not to be removed or bypassed under any circumstances.

33.1.2.1   Mechanical vibration of a tool should be considered and could contribute to

hand disabilities, due to vibration. Use vibration dampening gloves when

possible, and take intermitent breaks to lessen vibrational impact.

33.1.2.2   Mechanical dangers can also include material flying off of the object being

worked upon. Use safety glasses and extra safety shielding when this is a

possibility.

33.1.2.3   Great care must be taken not to have loose clothing or hair hanging close to

where power tools are being used. These can be instantly captured and drawn

into any rotating parts, reciprocating arms, moving belts, meshing gears,

shearing blades, or cutting teeth. Long hair must be secured, and loose

clothing covered or tied to prevent body parts being pulled into dangerous

areas.

33.1.3  Thermal hazards can be present when working with hand or power tools, especially on

material being worked on. Tool surfaces can also become hot as they are drilled or cut

and care should be exercised around these surfaces until they cool.

SECTION 34: BLOODBORNE PATHOGENS

34.1       This section applies to possible occupational exposure to blood or other potentially infectious

material. Blood has long been recognized as a source of pathogenic microorganisms that may

present a risk to individuals who are exposed during performance of their work assignments.

34.1.1  Typical bloodborne hazards include Viral Hepatitis B, Hepatitis C, Acquired Immune

Deficiency Syndrome (AIDS), Babesiosis, Arboviral infections, Relapsing Fever,

Creutzfeld-Jakob Disease, Viral Hemorrhagic Fever, Human T-lymphotrophic Virus Type

I, etc.

34.1.2  Though rare, exposure to blood at LD is a distinct possibility. Minor cuts and

abrasions can occur from sharp edges left by clipped component leads, jagged metal

edges, paper cuts, box cutting, nose bleeds, etc. Those providing first aid and workers

handling printed circuit boards or other material that may have caused a cut to another

person are at contamination risk. Even office workers can be at risk.

34.1.3  Emergency situations such as fire, earthquake, or a dropped chemical bottle can

greatly increase the possibility of cut and abrasion hazards in the workplace. Broken

glass, falling equipment, jagged metal, etc. must now all be considered as possible life

threatening situations. Scenes of accident or trauma are especially hazardous for

exposure to bloodborne pathogens.

34.1.4  OSHA has concluded that employees who have a reasonably anticipated skin, eye,

or mucous membrane contact with blood or other potentially infectious materials that

may result from the performance of the employee’s duties (“occupational exposure”) are

at risk of infection by bloodborne pathogens.

34.2       In order for employees to be protected from actual bloodborne pathogen exposure, protective

measures must be instituted. These measures include exposure control provisions to prevent

cuts and abrasions in the workplace and actions to be taken should a blood contamination

incident occur.

34.2.1  The Safety Manager will survey the workplace and determine those individuals at

higher risk of cuts and abrasions. For instance, those working around sharp metal

edges in the production area clipping component leads or handling printed circuit

assemblies could reasonably be expected to be at greater risk for cuts and abrasions

and would receive specialized training in the hazards present in their work areas and

the use of personal protective equipment, and what to do should an incident involving

blood take place.

34.2.1.1  Contractors, janitors, etc. will received specialized training in bloodborne

pathogens by the Safety Manager.

34.2.2   The work area will be inspected by employees prior to starting their assignments,

looking for any cut or abrasion hazards present. Steps should be taken to lower the

hazard by moving items to safer locations, installing protective barriers or shields, or

by putting on personal protective equipment such as heavy gloves, etc. Some tasks

have intrinsic hazards and can not be made any less hazardous. Employees

should be aware of and respect these situations.

34.3        If a cut, abrasion, nose bleed, etc. should ever occur, the following steps will be taken:

34.3.1  Bleeding will be brought under control by the victim, if possible, by placing paper or

cloth towels directly over the wound and pressing down hard. If a first responder is

controlling the bleeding, non-permeable gloves and protective eyewear must first be

put on before any action is taken. The Safety Manager must be notified no matter how

small  an incident involving the spilling of blood, to assess the hazards of the situation

and transport the victim to medical help if applicable.

34.3.2  Blood contaminated material that was utilized to control bleeding will be placed in a

plastic bag marked “bio-hazard” and given to the Safety Manager for disposal.

Protective equipment such as gloves and eyewear must always be worn when

handling any blood contaminated material.

34.3.3  The item that caused the cut or abrasion must be decontaminated to remove all

traces of blood residue for the safety of other workers. It is the responsibility of the

supervisor of the area where the incident occurred to see to it that this is done. The

material will be cleaned by swabbing the contaminated area with a 70% Isopropyl

Alcohol solution and disposing of the items in a bag marked “bio-hazard”. Protective

equipment such as gloves and eyewear must always be worn when handling any

blood contaminated surface.

SECTION 35: LASERS

35.1       Note: LD does not currently use high power lasers in its operations.  If using a laser pointer,

follow the manufacturers warnings included with the pointer.  Care must be taken to avoid

pointing into a person’s eyes or at a surface that could reflect the laser into a person’s

eyes.

SECTION 36: HAZARD COMMUNICATION STANDARD

36.1        Per 29 CFR 1910.1200, LD will provide employees with information regarding the physical

hazards and possible health effects relating to materials utilized in the workplace.

36.2        The workplace will be evaluated on a annual basis for the existence of hazardous substances

and suspected harmful physical agents.

36.2.1  A list of all chemicals on site will be provided by LD to the City of Provo on an annual

basis, fulfilling provisions of SARA Title III, commonly known as the “Community

Right to Know Act.”

36.2.2  No chemical will be allowed on site without a Material Safety Data Sheet first being

submitted to the Safety Manager. Purchasing must submit all chemical requests to the

Safety Manager for approval before a purchase order is issued and an order is placed.

The Safety Manager will not give approval until a current Material Safety data Sheet is

received from the manufacturer.

36.2.2.1  Chemical samples will not be allowed on LD premises until a

Material Safety Data Sheet is delivered to the Safety Manager.

36.2.3  A Material Safety Data Sheet book is available in Document Control and the

Safety Managers office for inspection by employees.

36.2.3.1  Before being allowed to work, all employees will be given training on the

hazardous materials utilized on the job site and how to read and interpret

Material Safety Data Sheets. Records of this training will be kept at the Safety

Manager’s office.

36.2.3.2  Current employees will receive annual training updates and as new

substances are introduced into the workplace to keep them aware of chemical

hazards. It is the supervisor’s responsibility to ensure that employees given

new assignments receive updated training. The Safety Manager must be

notified on any new job responsibilities and evaluate the necessary additional

training.

*** WARNING***

EMPLOYEES ARE NOT TO BEGIN WORKING  WITH ANY CHEMICAL

UNTIL THEY HAVE RECEIVED TRAINING REGARDING THE HAZARDS

ASSOCIATED WITH THE SPECIFIC CHEMICAL. CONTINUED

EMPLOYMENT AT LD IS CONTINGENT ON FOLLOWING

THIS DIRECTIVE.

36.3.2    A written copy of the LD Hazardous Waste Communication also known as

“Right To Know Program” is available to all employees for review.

36.3.3    Employees may request a personal copy of any MSDS sheet by submitting Form

PE2016-7, the “MSDS Request Sheet”, to the Safety Manager.

36.4         If one of the four physical agents listed below is generated by equipment or is present in a work

area, signs to this effect will be posted by the Safety Manager.

- Heat

- Noise

- Ionizing radiation

- Non-ionizing radiation

36.4.1    Employees that work in areas with any physical agents listed above will receive

specialized training in the effects of these hazards and how they can protect

themselves when working around these hazards.

INCIDENT REPORT INSTRUCTIONS

- If the incident requires medical attention beyond first aid administered at the site the following procedure should be followed while administering to the needs of the employee:

Notify Human Resources (General Manager) prior to taking the employee to a facility in non-emergency situations.

Incidents that are minor in nature and are not “life threatening” but do require medical attention such as stitches, serious scrapes, abrasions, burns, insect bites etc. should be treated at a work medical facility or insta care facility.  There are two locations in the area:

Orem Work Med 			Springville Work Med

830 N. 980 W.				385 So. 400 E

801-724-4000				801-491-6400

M-F 8AM to 5PM			M-F 8AM to 5PM

Incidents that are more serious in nature such as broken bones, serious bleeding, airway problems, heart attacks or other heart related issues, loss of consciousness, seizures, etc. should be treated at the:

Utah Valley Regional Medical Center (UVRMC)

1034 N 500 W

Provo, UT 84604

Transportation to these facilities should be by the supervisor in less acute situations and by ambulance when necessary.

- Supervisors are required to fill out and use the Incident Report to document any unusual or significant incident such as: accident, injury, near miss, fire, explosion, fight, intruder, power loss, serious equipment failure, significant storm damage, etc.

- The Incident Report must be completed by the end of the shift on which the incident occurred or as soon as practicable after the incident.

- Employee signatures are required on the Incident Report.

- Forward completed report to Human Resources.

Items #1 - #11: leave no blanks.

If a question isn’t applicable, write in “ N/A”.

If there were no witnesses (#4), write in “None”.

For #7: “Exact Location” - be specific about exactly where in the Department the incident occurred.

- “Outcome” information at bottom will be completed by the Human Resources Department.

INCIDENT INVESTIGATION DIRECTIONS

1.	Investigate any incident with the following consequences:

a.	An employee was injured.

b.	It was a “near miss”. There was no injury but it was a serious accident that could have caused injuries. The incident warrants investigation to prevent it from happening again.

c.	There was significant damage.

2.	Investigate any incident (e.g. fire, storm damage, equipment failure, intruder) which resulted in an injury or damage or could have had serious consequences and warrants investigation.

3.	Talk to the affected employee and/or witness. Get the facts.

4.	Look for more than one cause.  Look for contributing causes.

5.	Contributing causes are circumstances or conditions that contributed to or led to the unsafe condition or unsafe act.  Explain why the unsafe condition or unsafe act occurred.

6.	A corrective action is required for each cause.

UNSAFE BEHAVIORS

| Possible Unsafe Acts | Possible Corrective Actions |
| --- | --- |
| - Employee did not do the job correctly. - Employee failed to follow instructions. - Employee didn’t use proper safety procedures. - Employee committed a blatant unsafe act. - Employee did something he shouldn’t have. - Employee wasn’t wearing the right PPE. | - Give job instruction & training. - Re-instruct employee in correct way to do the task. - Establish proper work method. - Change work method. - Review proper safety procedures with employee - Establish proper safety procedures for the task. - Change safety procedures for the task. - Disciplinary action. |

UNSAFE CONDITIONS

| Possible Unsafe Conditions | Possible Corrective Actions |
| --- | --- |
| - Unsafe machinery or equipment. - Unsafe tool, materials or item. - A safety hazard needs better safeguarding. - Defective or damaged part is creating a hazard. | - Fix, replace or repair unsafe item. - See maintenance about fixing the problem. - Write Work Order. - Purchase item |

Fire Extinguisher / Emergency Light Inspection

FORM PE2016-2, REV B,  02/24/12

Prepared by  _____________________________Title_______________________Date  ____/____/____

South Administration Building

Circle problem type below and mark the location on the following diagram.

Δ

Δ

Δ

Δ

Δ

Δ

Exit

Exit

Exit

North

☼

S1

☼

S2

☼

S3

☼

S9

☼

S10

☼

S11

☼

S8

☼

S7

☼

S6

☼

S4

☼

S5

_☼_ = Emergency Lighting                             _Δ_  = Fire Extinguishers

Fire extinguishers found to be okay:   Yes    No              Emergency lights found to be okay:    Yes     No

_Δ_  FIRE EXTINGUISHERS:     Missing         Blocked       Expired        Tag Missing        Discharged

_☼_  EMERGENCY LIGHTS:      Non-Operational        Bulb Burned Out        Insufficient Battery Life

Notes:______________________________________________________________________________ ___________________________________________________________________________________ ___________________________________________________________________________________ ___________________________________________________________________________________

Service was called on____/____/____                                   Repairs were completed on____/____/____

Fire Extinguisher / Emergency Light Inspection      Page 2

Production Building

Circle problem type below and mark the location on the following diagram.

North

Δ

Δ

Δ

Δ

Δ

Δ

Δ

Δ

Δ

Δ

☼

N1

☼

N5

☼

N6

☼

N7

☼

N4

☼

N2

☼

N3

Fire extinguishers found to be okay:     Yes     No          Emergency lights found to be okay:     Yes     No

_Δ_  FIRE EXTINGUISHERS:     Missing         Blocked       Expired        Tag Missing        Discharged

_☼_  EMERGENCY LIGHTS:      Non-Operational        Bulb Burned Out        Insufficient Battery Life

Notes:______________________________________________________________________________ ___________________________________________________________________________________ ___________________________________________________________________________________ ___________________________________________________________________________________

Service was called on____/____/____                                   Repairs were completed on____/____/____

Ventilation Inspection                                 FORM PE2016-3, REV A, 03/31/08

Prepared by  _________________________  Date  ____/____/____Title  _________________________

Exhaust hood location (building)                    _____       Exhaust hood operational?     Yes     No

Exhaust hood number                                    _____       Velometer reading at front of work surface  _____

Height of shield (if any) from work surface     _____        Velometer reading at rear of work surface  _____

Repairs needed?    Yes    No            If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the hood

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Exhaust hood location (building)                    _____       Exhaust hood operational?     Yes     No

Exhaust hood number                                    _____       Velometer reading at front of work surface  _____

Height of shield (if any) from work surface     _____        Velometer reading at rear of work surface  _____

Repairs needed?    Yes    No            If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the hood

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Exhaust hood location (building)                    _____       Exhaust hood operational?     Yes     No

Exhaust hood number                                    _____       Velometer reading at front of work surface  _____

Height of shield (if any) from work surface     _____        Velometer reading at rear of work surface  _____

Repairs needed?    Yes    No            If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the hood

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Exhaust hood location (building)                    _____       Exhaust hood operational?     Yes     No

Exhaust hood number                                    _____       Velometer reading at front of work surface  _____

Height of shield (if any) from work surface     _____        Velometer reading at rear of work surface  _____

Repairs needed?    Yes    No            If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the hood

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Safety Inspection Log                              FORM PE2016-4, REV A, 03/31/08

Prepared by  ______________________________________________________  Date  ____/____/____

Building  _______________   Location  _______________ Copy given to area supervisor      Yes      No

HAZARDOUS PHYSICAL CONDITIONS

_____	trip hazards					_____	guards removed from equipment

_____	exposed electrical wires				_____	unlabeled chemical containers

_____	blocked isles					_____	gas cylinders not secured

_____	racks not secured to wall			_____	hot surfaces not labeled

_____	overloaded electrical cords / fire hazards		_____	heavy items high on shelves

_____	safety gear needed for situation but not worn	_____	misuse of equipment

_____	safety interlocks bypassed			_____	overhead hazard not marked

details of above situations or other hazardous physical conditions: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

ENVIRONMENTAL CONCERNS

_____	lights burned out				_____	fume buildup / poor ventilation

_____	uncomfortable temperature			_____	excessive noise

_____	ergonomic problems in specific work areas	_____	clutter

details of above situations or other environmental concerns:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

REGULATORY CONCERNS

_____	emergency exits not marked or blocked		_____	fire extinguisher blocked

_____	storage within 18” of a sprinkler head		_____	fire extinguisher expired

_____	storage within 36” of electrical breaker box	_____	exit isles blocked

_____	electrical breakers not labeled             		_____	eye wash / showers blocked

details of above situations or other regulatory concerns:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Hazardous situations were:        immediately taken care of

   assigned to the area supervisor to rectify by  ____/____/____                    Followup  ____/____/____

Safety Eye Wash Inspection                     Form PE2016-5, REV A, 03/31/08

Prepared by  _____________________________  Date  ___/___/___Title  ________________________

Eye wash location (building)           _____________        Eye wash operational?     Yes     No

Unit number                                                   _____         Eye wash bottle full?     Yes     No

Repairs needed?    Yes    No         If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the fixture

Notes  ______________________________________________________________________________

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Eye wash location (building)           ____________          Eye wash operational?     Yes     No

Unit number                                                   _____         Eye wash bottle full?     Yes     No

Notes  ______________________________________________________________________________

Repairs needed?    Yes    No         If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the fixture

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Eye wash location (building)           ____________          Eye wash operational?     Yes     No

Unit number                                                   _____         Eye wash bottle full?     Yes     No

Notes  ______________________________________________________________________________

Repairs needed?    Yes    No         If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the fixture

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Eye wash location (building)           ____________          Eye wash operational?     Yes     No

Unit number                                                   _____         Eye wash bottle full?     Yes     No

Notes  ______________________________________________________________________________

Repairs needed?    Yes    No         If yes,  place an “OUT OF SERVICE-DO NOT USE” sign on the fixture

Service was called on ____/____/____                                  Repairs were completed on  ____/____/____

Medical History Questionaire              FORM PE2016-6, REV A, 03/31/08

For a qualified physician to properly assess the ability of an employee to wear a respirator, a medical history for each wearer is required. This questionaire will aid in providing that history. In addition to this questionaire, the reviewing physician may require a physical exam and a pulmonary function test.

Name  ___________________________________________  Social Security Number  ____/____/_____

Home Address  _____________________  City  _______________  State  _____   Zip  ______________

Phone  (___)____-_______   Job Title  _______________________  Age  _____   Sex      Male     Female

Employee/Payroll No.  ______________  Department  _______________  Supervisor  _______________

Type of exposure:         organics       dust       acid       fume      toxic      oxygen deficient

Respirator recommended  _______________________________________________________________

PLEASE FILL OUT THE QUESTIONAIRE AS ACCURATELY AS POSSIBLE     -     CIRCLE ANSWER

Yes     No     Do you have insulin dependent diabetes or mellitus?

Yes     No     Do you have epilepsy or have suffered a grand-mal or petit-mal seizure in the last 6 months?

Yes     No      Has your health changed for the worst over the last year?

Yes     No      Do you use medications?  Please list:  __________________________________________

_________________________________________________________________________

_________________________________________________________________________

Yes     No      Have you ever had a punctured ear drum?

Yes     No      Do you have skin allergies?

Yes     No      Do you have impared or a non-existent sense of smell?

Yes     No      Do you have emphysema?

Yes     No      Do you have chronic pulmonary obstructive disease (CPOD)?

Yes     No      Do you have asthma?

Yes     No      Do you or have you ever had pneumoconiosis (dust related disease)?

Yes     No      Is there any evidence, to your knowledge of reduced pulmonary (lung) function?

Yes     No      Do you have a history of heart attack, stroke, or other heart problems?

Yes     No      Do you have uncontrolled hypertension or untreated hypertension?

Yes     No      Do you have a problem with walking up two flights of stairs at a rapid pace?

Medical History Questionaire                                                              Page 2

Yes     No      Do you have other breathing problems?  Write your comments on the back of this page

Yes     No      Have you ever experienced difficulty in breathing when wearing a respirator?

Yes     No      Have you ever experienced claustrophobia when wearing a respirator?

Yes     No      Are there any other medical conditions that you feel could affect the safe use of a respirator?

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

Employee signature  ________________________________________________  Date  ____/____/____

DO NOT WRITE BELOW THIS LINE

I have examined this questionaire to determine the physical fitness required to use respiratory protective equipment by _______________________(employee name).

The following restictions shall apply:  ____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

At this time, I can find no physical reason to prohibit the above named individual from participating in occupational tasks that may require the use of air purifying or air supplied respirators.

Physician  ________________________________________________________  Date  ____/____/____

Based on physical limitations and/or medical history, I cannot give my approval for the aboved named individual to participate in occupational tasks that may require the use of air purifying or air supplied respirators.

Physician  ________________________________________________________  Date  ____/____/____

Safety Manager review  __________________  Date  ____/____/____    Employee told  ____/____/____

MSDS Request Sheet                          FORM PE2016-7, REV A, 03/31/08

Name  ___________________________________________________________  Date  ____/____/____

Department  __________________________________________  Supervisor  _____________________

I am requesting a copy of the Material Safety Data Sheet for the following chemcial substance(s):

chemical  __________________________________  vendor  ___________________________________

chemical  __________________________________  vendor  ___________________________________

chemcial  __________________________________  vendor  ___________________________________

chemical  __________________________________  vendor  ___________________________________

chemical  __________________________________  vendor  ___________________________________

chemical  __________________________________  vendor  ___________________________________

chemical  __________________________________  vendor  ___________________________________

chemcial  __________________________________  vendor  ___________________________________

Signature  __________________________________

I acknowledge that I have received the MSDS sheet(s) that I have requested above.

Signature  __________________________________  Date  ____/____/____

The highlighted MSDS which was requested is a commercial product and an MSDS sheet may not be required / available. The supplier will be contacted to provide either a copy of an MSDS sheet or a detailed ingredient and hazard listing in lieu of an MSDS sheet. Until this arrives, use the product label to provide information on ingredients and any hazards involved with using the product.

Supervisor  _________________________________  Date  ____/____/____

Safety Manager  _____________________________  Date  ____/____/____

Ladder Inspection        	             Form PE2016-8, REV A, 02/24/12

Prepared by  _____________________________  Date  ___/___/___Title  ________________________

Building   ____________    Ladder # _______

Stored properly?     Yes    No

Ladder free of grease, oil, other contaminates    Yes    No

If no, place an “OUT OF SERVICE-DO NOT USE” sign on the ladder.

Notes  ______________________________________________________________________________

Clean up completed on  ____/____/____

Is ladder in good repair and stable?     Yes     No

Notes  ______________________________________________________________________________

Repairs were completed on  ____/____/____

Building   ____________    Ladder # _______

Stored properly?     Yes    No

Ladder free of grease, oil, other contaminates    Yes    No

If no, place an “OUT OF SERVICE-DO NOT USE” sign on the ladder.

Notes  ______________________________________________________________________________

Clean up completed on  ____/____/____

Is ladder in good repair and stable?     Yes     No

Notes  ______________________________________________________________________________

Repairs were completed on  ____/____/____