Required Software:

- U-WAVEPAK

- MS Excel or Word

U-WAVE TC Wireless Data Transmitter and Connection Unit

U-WAVE TM

Wireless Data Transmitter and Connection Unit

Wireless Data Receiver

Figure 1 – U-WAVE System

# Activate System

      - Click the Mitutoyo U-WAVEPAK icon (“M”) on the desktop or taskbar.

Figure 2

      - Click [Data I/F start] on the U-WAVEPAK menu screen.

Figure 3

>> [U-WAVEPAK Data I/F] starts, and U-WAVEPAK icon (“M”) is displayed in the Windows taskbar.

U-WAVEPAK

activated

Figure 4

      - The system is ready for use.

# Collect Measurement Data

- Open the desired data sheet (Excel spreadsheet shown below as an example).

- Select the cell to import measurement data into.

Figure 5

- Measure part with SPC mics or calipers.  With the Power (green LED) indicator of the U-WAVE receiver lit, press the [DATA] button on the U-WAVE transmitter (on SPC mics or calipers) to import the reading into the selected cell.  Once the U-WAVE transmitter successfully communicates with the receiver, the LED indicator on the transmitter blinks in green.

NOTE:  Be sure the correct cell is selected in the worksheet before measuring a part.  It is also advisable to periodically save the data sheet to prevent loss of data in case of a sudden malfunction (power outage, network or computer problems, etc.).

# Exit System

- Click the U-WAVEPAK icon (“M”) on the Windows taskbar after final measurement data has been imported and saved.

- Click [Stop data I/F], and then click [Exit data I/F].

Figure 6

>> The U-WAVEPACK menu screen is displayed.

- Click [Exit].

Figure 7

>> U-WAVEPAK exits.

APPENDIX A - Install U-WAVE Software and Hardware

- Install U-WAVE Receiver

(Video instructions may be found here: https://www.youtube.com/watch?v=WzDmZT_9KSc)

- To start, you need 4 things:

- U-WAVE receiver

- Supplied USB cable

- Local computer

- U-WAVEPAK software

- Connect the supplied USB cable between the back of the receiver and USB port in the computer.  Note, the power light on the receiver is not illuminated.

- Load U-WavePak software from installation disc or from a pre-installed location on the computer or network directory (e.g. R:\Crystals\Mitutoyo U-Wave).  Click on U-WavePak folder.

- Click on Setup directory, then double-click on Setup.exe to begin installation.

- Select language, then click OK to accept.

- The installation welcome screen will appear.  Click Next to continue.

- Accept license agreement and then click Next.

- Indicate where software will be installed on computer. Location may be different from that shown in the following image.

- Click Next.

- The Start Menu folder will display.  Click Next to continue with the wizard.

- Check to create a desktop icon, then click Next to continue.

- A summary of previous pages in the wizard will display.

- Click Install to begin the installation.

- Once complete, click Finish to exit the installer.

- Note, the power light is still not illuminated.

- Install Drivers for U-WAVE Receiver

- In the computer Control Panel, select Device Manager.

- Two drivers will need to be installed.  The first driver will be for the Mitutoyo U-WAVE found under the “Other devices” folder.

- Right-click “Mitutoyo U-WAVE” and select Update Driver Software.

- Select “Browse my computer for driver software”

- Browse to the folder that contains the driver software.  This is the same folder U-WavePak is installed in.

- Click on UWAVEPAK folder, the click OK.

- Click Next to continue.

- The drivers will install.

- Once drivers are loaded, click Close to return to the Device Manager.

- Notice the icon has changed next to the Mitutoyo U-WAVE device, which means the drivers were successfully loaded.

- Install the second driver, which is for the USB Serial Port.  Right-click USB Serial Port and select Update Driver Software.

- Select “Browse my computer for driver software”

- Notice the folder is still entered from the last driver installed.  Click Next.

- Click Close to return to Device Manager.

- Notice icon has changed.  Note, COM port may be different than what’s shown in image.

- Close out of Device Manager.

- For a new installation, lights on receiver will be begin blinking, alternating red and green.  This indicates the receiver needs to be initialized.

- Launch U-WAVEPAK.

- Click Setup start to begin setting up the receiver.

- The following will display on a new installation to notify the receiver has not yet been initialized.

- Click Yes.

- The default setting for the receiver will display.  Unless a reason to change, click OK to continue.

- Press Yes to initialize the receiver.

- The settings from the previous screen ae now active.

- Pair U-WAVE Transmitter and Receiver

(Video instructions may be found at https://www.youtube.com/watch?v=BNVLmyG4D5g)

- Note the device IDs on the side of the transmitter and the bottom of the receiver.

- Launch U-WAVEPAK software.

- Click “Setup start” to begin pairing the transmitter to the receiver.

- Note U-WAVE-R in top section shows the receiver currently plugged into the computer.  U-WAVE-T section shows transmitters currently paired to the receiver.  Note there are no paired transmitters in the image.

- To add a transmitter, click the Add button.

- Enter the transmitter’s device ID.

- Enter the desired channel.  The default is the first available number.  In this example, it’s “00”.

- The Group ID and band ID are linked to the receiver.  Do not change these settings.

- Click OK to continue.

- A confirmation screen will appear.  This means the receiver is ready to receive data from the newly added transmitter for the first time.

- Click “Yes”.

- Make sure the measuring device (with transmitter) is ready to send data, and there is a reading on the device.

- Press and hold the data send button on the transmitter for about 1 second.  Notice the “r”  in the S column has changed to a “c”, which means the gage is connected.

- Press the data send button on the transmitter.  Note on the bottom of the U-WAVE software that the data has been transferred.

- The transmitter is now connected and you can exit the U-WAVEPAK set-up.

- Click Exit setup.

- Unpair U-WAVE Transmitter & Receiver

(Video instructions may be found at https://www.youtube.com/watch?v=K5VSMplDiIg)